- Tujuan/Objective

Untuk memastikan tata cara pemeriksaan bahan baku tambahan Magnesium Stearate dilakukan dengan baik dan benar sesuai spesifikasi yang berlaku.

To guarantee that the method for inspecting Magnesium Stearate raw materials is followed appropriately and in accordance with applicable spesifications.

- Ruang Lingkup/Scope

Laboratorium PT. Kimia Farma, Tbk - Plant Banjaran.

PT Kimia Farma Tbk. - Plant Banjaran Laboratory.

- Penanggung Jawab/Responsibility

Analis Pengawasan Mutu bertanggung jawab melaksanakan pemeriksaan bahan baku Magenesium Stearate sesuai dengan instruksi kerja ini.

The Quality Control Analyst is in charge of inspecting Magnesium Stearate raw materials in line with the job instructions.

- Definisi/Definition

-

- Referensi/Reference

USP 2022;

PT. Kimia Farma.

- Alat dan Bahan/Equipments and Materials

  - Alat/Equipments :

| 1. | Timbangan Analitik/Analytical Balance | 13. | Buret/Burette |
| --- | --- | --- | --- |
| 2. | Gelas Kimia/Beaker Glass | 14. | Desikator/Desiccator |
| 3. | Tabung Reaksi/Test Tube | 15. | Corong Pisah/Separator |
| 4. | Labu Ukur/Volumetric Flask | 16. | Botol Timbang/Weighing Bottle |
| 5. | Oven | 17. | Magnetic Stirrer |
| 6. | Pipet Ukur/Measuring Pipette | 18. | Erlenmeyer/Erlenmeyer flask |
| 7. | Inductively Coupled Plasma (ICP) | 19. | Erlenmeyer Berasah/Conical Flask |  |
| 8. | Vortex | 20. | Waterbath |  |
| 9. | Gelas Ukur/Measuring Cylinder | 21. | Tabung Nessler/Nessler Tube |  |
| 10. | Penjepit Cawan/Crucible Tong | 22. | Kertas Saring/Filter Paper |  |
| 11. | Kromatografi gas/Gas chromatography | 23. | Alat Refluks/Reflux apparatus |  |
| 12. | Kolom G16; 0,32 mm x 30 m; 0,5 µm/ Column G16; 0,32 mm x 30 m; 0,5 µm |  |  |  |

  - Bahan/Materials :

| 1. | Air Murni/Purified Water | 14. | Amonium Hidroksida P; 6 N P; 6 N Ammonium Hydroxide |
| --- | --- | --- | --- |
| 2. | Air Murni Bebas CO2 CO2 - free Water | 15. | Natrium Fosfat Dibasa LP Dibasic Sodium Phosphate TS |
| 3. | Asam Klorida 3 N; 1 N; 0,1 N LV; 0,02 N/ 3 N; 1 N; 0,1 N LV; 0,02 N Hydrochloric Acid | 16. | Eter/Ether |
| 4. | Amonium Klorida P/Ammonium Chloride | 17. | Perak Nitrat 0,1 N/0,1 N Silver Nitrate |
| 5. | Butil Alkohol/Butyl Alcohol | 18. | EDTA 0,1 M LV/0,1 M EDTA VS |
| 6. | Zink Sulfat 0,1 M LV/0,1 M Zinc Sulfate VS | 19. | Eter Bebas Peroksida Peroxide - free Ether |
| 7. | ICP Multi-element Standard Solution IV 100 ppm | 20. | Natrium Hidroksida 0,1 N LV 0,1 N Sodium Hydroxide VS |
| 8. | Alkohol Dehidrat Dehydrated Alcohol | 21. | Etanol/Ethanol |
| 9. | Eriochrome Black T | 22. | Asam Nitrat P; 2 N P; 2 N Nitric Acid |
| 10. | Asam Sulfat 0,02 N 0.02 N Sulfuric Acid | 23. | Barium Klorida LP Barium Choride TS |
| 11. | Amonium Karbonat LP Ammonium Carbonate TS | 24. | Bromtimol Biru LP Bromothymol Blue TS |
| 12. | Boron Triflorida/Boron Triflouride | 25. | n-Heptan/n-Heptane |
| 13. | Natrium Hidroksida jenuh/ Saturated Sodium Hydroxide |  |  |

- Instruksi Kerja/Work Instruction

  - Pemerian/Description

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

Amati secara visual (Organoleptis).

Visually observe (Organoleptic).

  - Kelarutan/Solubility

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Kelarutan dalam Air/Solubility In Water

- Larutkan 10 mg zat uji dengan lebih dari 100 ml air murni dalam gelas kimia.

Dissolve 10 mg of sample substance with more than 100 mL of pure water in a beaker.

- Amati pemerian larutan, larutan jernih.

Observe the description of the solution, there is clear solution.

- Kelarutan dalam Etanol/Solubility In Ethanol

- Larutkan 10 mg zat uji dengan lebih dari 100 ml etanol dalam gelas kimia.

Dissolve 10 mg of sample substance with more than 100 mL of ethanol in a beaker.

- Amati pemerian larutan, larutan jernih.

Observe the description of the solution, there is clear solution.

- Kelarutan dalam Eter/Solubility In Ether

- Larutkan 10 mg zat uji dengan lebih dari 100 ml ether dalam gelas kimia.

Dissolve 10 mg of sample substance with more than 100 mL of ether in a beaker.

- Amati pemerian larutan, larutan jernih.

Observe the description of the solution, there is clear solution.

  - Identifikasi (R)/Identification (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

      - Uji Identifikasi A/Identification test A

- Timbang 5 gram zat uji, masukkan ke dalam labu Erlenmeyer.

Weigh 5 grams of the sample substance, put it in an erlenmeyer flask.

- Tambahkan 50 ml Eter, 20 ml HNO3 2 N dan 20 ml Air Murni.

Add 50 mL of  Peroxide - free Ether, 20 mL of  HNO3 2 N, and 20 mL of Purified Water.

- Panaskan di atas Waterbath pada suhu 105C selama 30 menit.

Heat it on a Waterbath at 105C for 30 minutes.

- Dinginkan, lalu masukkan kedalam corong pisah.

Allow to cool, then put into a separator.

- Kocok corong pisah dan biarkan lapisan terpisah.

Shake the separator and allow the layers to separate.

- Pisahkan lapisan air ke dalam labu erlenmeyer lain.

Separate the aqueous layer into another erlenmeyer flask.

- Ekstrak lapisan Eter dengan 4 ml Air Murni.

Extract the Ether layer with 4 mL of Purified Water.

- Kocok corong pisah dan biarkan lapisan terpisah.

Shake the separator and allow the layers to separate.

- Pisahkan lapisan air dan gabungkan dengan lapisan air yang pertama.

Separate the water layers and add them to the first water layer.

- Masukkan ke dalam corong pisah yang lain.

Put it in another separator.

- Cuci lapisan air dengan menambahkan 15 ml Eter Bebas Peroksida dan mengocok corong pisah tersebut.

Wash off the aqueous layer by adding 15 mL of Peroxide - Free Ether and shaking the separator.

- Pisahkan lapisan air ke dalam labu ukur 50 ml.

Separate the aqueous layer into a 50 mL volumetric flask.

- Tambahkan Air Murni hingga tanda batas volume, kocok hingga homogen

(Larutan A).

Add Purified Water to volume, shake until homogeneous (Solution A).

- Pipet 1 ml Larutan A, masukkan ke dalam tabung reaksi.

Pipette 1 mL of Solution A into a test tube.

- Tambahkan Ammonium Klorida P ke dalam larutan.

Add Ammonium Chloride to the solution.

- Netralkan dengan Ammonium Karbonat LP, akan samar-samar terbentuk endapan.

A slightly hazy precipitate when neutralized with ammonium carbonate TS.

- Setelah penambahan Natrium Fosfat Dibasa LP, akan terbentuk endapan kristal putih yang tidak larut dalam Ammonium Hidroksida 6 N.

On the subsequent addition of dibasic sodium phosphate TS, a white, crystalline precipitate, which is insoluble in 6 N ammonium hydroxide, is formed.

      - Uji Identifikasi B/Identification test B

Waktu retensi puncak yang sesuai dengan Asam Stearat dan Asam Palmitat dari Larutan uji sesuai dengan Larutan Uji kesesuaian sistem, seperti yang diperoleh di Uji Kandungan Relatif Asam Stearat dan Asam Palmitat.

The retention times of the peaks corresponding to stearic acid and palmitic acid of the Sample solution correspond to those of the System suitability solution, as obtained in the test for Relative Content of Stearic Acid and Palmitic Acid.

  - Klorida/Chloride

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Larutan Baku/Standard Solution

- Pipet 1,4 ml HCl 0,02 N, masukkan ke dalam tabung nessler.

Pipette 1.4 mL of 0.02 N HCl, put it in a nessler tube.

- Larutan Uji/Sample Solution

- Pipet 10 ml Larutan A, masukkan ke dalam tabung nessler.

Pipette 10 mL of Solution A, put it in a nessler tube.

- Pengamatan/Observation

- Ke dalam masing-masing tabung Nessler yang berisi larutan baku dan larutan uji, tambahkan Air Murni hingga total volume 30 - 40 ml.

Into each Nessler tube containing the standard solution and the test solution, add purified water to a total volume of 30 - 40 mL.

- Tambahkan juga 1 ml HNO3 2 N dan 1 ml AgNO3 0,1 N.

Also add 1 mL of 2 N HNO3 and 1 mL of 0.1 N AgNO3.

- Tambahkan Air Murni hingga volume 50 ml, kocok hingga homogen.

Add Purified Water to a volume of 50 mL, mix until homogeneous.

- Biarkan selama 5 menit di tempat yang jauh dari sinar matahari langsung.

Allow to stand for 5 minutes protected from direct sunlight.

- Amati pemerian larutan, lalu bandingkan kekeruhan larutan uji dengan kekeruhan larutan baku.

Observe the description of the solution, then compare the turbidity of the sample solution with the turbidity of the standard solution.

  - Sulfat/Sulfate

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Larutan Baku/Standard Solution

- Pipet 3 ml H2SO4 0,02 N, masukkan ke dalam tabung nessler.

Pipette 3 mL of 0.02 N H2SO4, put it in a nessler tube.

- Larutan Uji/Sample Solution

- Pipet 6 ml Larutan A, masukkan ke dalam tabung nessler.

Pipette 3 mL of Solution A, put it in a nessler tube.

- Pengamatan/Observation

- Ke dalam masing-masing tabung Nessler yang berisi larutan baku dan larutan uji, tambahkan Air Murni hingga total volume 30 - 40 mL.

Into each Nessler tube containing the standard solution and the test solution, add purified water to a total volume of 30 - 40 mL.

- Tambahkan juga 1 ml HCl 3 N dan 3 ml Barium Klorida LP.

Also add 1 mL of 3 N HCl and 1 mL of Barium Chloride TS.

- Tambahkan Air Murni hingga volume 50 ml, kocok hingga homogen.

Add Purified Water to a volume of 50 mL, mix until homogeneous.

- Biarkan selama 5 menit.

Allow to stand for 5 minutes.

- Amati pemerian larutan, lalu bandingkan kekeruhan larutan uji dengan kekeruhan larutan baku.

Observe the description of the solution, then compare the turbidity of the sample solution with the turbidity of the standard solution.

  - Kadar (R)/Assay (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Buffer

- Timbang 5,4 gram Ammonium Klorida, masukkan ke dalam gelas kimia.

Weigh 5.4 grams of Ammonium Chloride, put it in a beaker glass.

- Tambahkan 20 ml Ammonium Hidroksida P.

Add 20 mL of Ammonium Hydroxide.

- Encerkan dengan 80 ml Air Murni, aduk hingga homogen.

Dilute with 80 mL of Purified Water, stir until homogeneous.

- Pemeriksaan/Inspection

- Timbang 500 mg zat uji, masukkan ke dalam labu Erlenmeyer.

Weigh 500 mg of the sample substance, put it in an erlenmeyer flask.

- Tambahkan 50 ml campuran Butil Alkohol : Alkohol Dehidrat (1: 1); 5 ml Ammonium Hidroksida P; 3 ml Buffer; 30,0 ml EDTA 0,1 M LV; dan 2 tetes Eriochrome Black T.

Add 50 mL of the mixture of Butyl Alcohol : Alcohol Dehydrate (1:1); 5 mL Ammonium Hydroxide; 3 mL of Buffer; 30.0 mL of 0.1 M Edetate Sodium VS; and 2 drops of Eriochrome Black T.

- Panaskan pada suhu 45 - 50oC hingga larutan jernih, dinginkan.

Heat at a temperature of 45 - 50oC until the solution is clear, allow to cool.

- Titrasi kelebihan EDTA dengan Zink Sulfat 0,1 M LV hingga larutan berubah warna dari biru menjadi ungu.

Titrate the excess Edetate Sodium with 0.1 M Zink Sulfate VS until the solution color changes from blue to violet.

- Lakukan titrasi blanko.

Perform a blank titration.

Setiap ml EDTA 0,1 M LV ekivalen dengan 2,431 mg Magnesium.

Each mL of 0.1 M Edetate Sodium VS is equivalent to 2.431 mg of Magnesium.

    - Perhitungan/Calculation :

C=Vb -Vu × M0,1 × 2,431Wu × 100%(100% -SP) × 100%

Keterangan/Note:

C	=	Kadar Magnesium (%)

Assay of Magnesium (%)

Vb	=	Volume Zink Sulfat 0,1 M LV untuk titrasi larutan blanko (ml)

Volume of Zinc Sulfate 0.1 M VS for titration of blank solution (mL)

Vu	=	Volume Zink Sulfat 0,1 M LV untuk titrasi larutan uji (ml)

Volume of Zink Sulfate 0.1 M LV for titration of sample solution (mL)

M	=	Molaritas Zink Sulfat LV (M)

Zink Sulfate VS Molarity (M)

0,1	=	Molaritas teoritis Zink Sulfat LV (M)

Theoretical molarity of Zink Sulfate VS (M)

2,431	=	Kesetaraan 1 ml EDTA 0,1 M terhadap Magnesium (mg)

Equivalence of 1 mL 0,1 M EDTA to Magnesium (mg)

Wu	=	Berat zat uji yang ditimbang (mg)

Weight of sample substance weighed (mg)

SP	=	Susut Pengeringan (%)

Loss on Drying (%)

  - Keasaman atau Kebasaan/Acidity or Alkalinity

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Timbang 1 gram zat uji, masukkan ke dalam Erlenmeyer Berasah.

Weigh 1 gram of the sample substance, put it in a conical flask.

- Tambahkan 20 ml Air Bebas CO2, didihkan dalam Waterbath selama 1 menit.

Add 20 mL of CO2-free water, boil in a water bath for 1 minute.

- Kocok terus-menerus,  dinginkan, kemudian saring.

Continuous shaking, cool, and filter.

- Pipet 10 ml filtrat, lalu tambahkan 0,05 ml Bromtimol Biru LP.

Pipette 10 mL of the filtrate, then add 0.05 mL of Bromtimol Blue LP.

- Jika larutan berwarna kuning, titrasi larutan dengan NaOH 0,1 N LV hingga berwarna biru.

If the solution is yellow, titrate the solution with 0.1 N NaOH VS until it becomes blue.

- Jika larutan berwarna biru, titrasi larutan dengan HCl 0,1 N LV hingga berwarna kuning.

If the solution is blue, titrate the solution with 0.1 N HCl VS until it becomes yellow.

- Catat volume peniter.

Record the volume of the titrant.

  - Susut Pengeringan (R)/Loss On Drying (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Panaskan botol timbang pada suhu 105oC di dalam Oven, selama 30 menit.

Heat the weighing bottle at a temperature of 105oC in the Oven, for 30 minutes.

- Dinginkan botol timbang dalam desikator selama 5 – 15 menit dan timbang seksama (Wo).

Allow to cool the weighing bottle in a desiccator for 5 – 15 minutes and weigh thoroughly (Wo).

- Bila zat uji berupa kristal besar, gerus secara cepat hingga ukuran partikel ± 2 mm.

If the test substance is big crystals, mill it quickly to a particle size of ± 2 mm.

- Timbang seksama 1 g zat uji, masukkan ke dalam botol timbang yang telah ditara (Wa).

Weigh carefully 1 g of the test substance, put it in a weighing bottle that has been weighed (Wa).

- Masukkan ke dalam oven dengan suhu 105oC.

Put it in the oven at 105oC.

- Keringkan selama 3 jam.

Dry for 3 hours.

- Dinginkan dalam desikator selama 5 - 15 menit, kemudian timbang (Wb).

Cool in a desiccator for 5 - 15 minutes before weighing (Wb).

    - Perhitungan/Calculation :

SP= Wa-WbWa-Wo × 100%

Keterangan/Note:

SP	= 	Susut pengeringan

Loss on drying (%)

Wa	= 	Berat botol timbang + zat uji sebelum pengeringan (gram)

Weight of weighing bottle + sample substance before drying (grams)

Wb	= 	Berat botol timbang + zat uji setelah pengeringan (gram)

Weight of weighing bottle + sample substance after drying (grams)

Wo	= 	Berat botol timbang kosong (gram)

Weight of empty weighing bottle (grams)

  - Logam Berat/Heavy Metals

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

- Pemeriksaan Timbal (Pb), Kadmium (Cd), dan Nikel (Ni)

Examination of Lead (Pb), Cadmium (Cd), and Nickel (Ni)

- Larutan Asam Nitrat 2,5%/Nitric Acid 2,5% Solution

- Ambil 500 ml air murni, masukkan ke dalam gelas kimia 1000 ml.

Fill a 1000 mL beaker halfway with 500 mL of fresh water.

- Tambahkan 38,5 ml Asam Nitrat P secara perlahan; aduk hingga homogen.

Slowly pour in 38.5 mL of Nitric Acid  and mix until smooth.

- Tambahkan 461,5 ml Air Murni; aduk hingga homogen.

Add 461.5 mL of Purified Water; stir until homogeneous.

- Larutan Baku Multi-element Induk/Multi-Element Standard Solution

- Pipet 2 ml ICP Multi-element Standard Solution IV 100 ppm, masukkan ke dalam labu ukur 20 ml.

Pipette 2 mL ICP Multi-element Standard Solution IV 100 ppm, placed in a 20 mL volumetric flask.

- Tambahkan larutan Asam Nitrat 2,5% hingga tanda batas volume; kocok hingga homogen.

To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Saring menggunakan syringe filter 0,2 µm.

Filter using a 0.2 µm syringe filter.

(Konsentrasi Pb, Cd, Ni: 10 ppm)/(Concentration of Pb, Cd, Ni: 10 ppm)

- Larutan Baku Multi-element 0,1 ppm/Multi-element standard solution 0.1 ppm

- Pipet 0,5 ml Larutan Baku Multi-element Induk; masukkan ke dalam labu ukur 50 ml.

Pipette 0.5 mL of Multi-element Standard Solution into a volumetric flask with a 50 mL capacity.

- Tambahkan larutan Asam Nitrat 2,5% hingga tanda batas volume; kocok hingga homogen.

To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Saring menggunakan syringe filter 0,2 µm.

Filter using a 0.2 µm syringe filter.

(Konsentrasi Pb, Cd, Ni: 0,1 ppm)/(Concentration of Pb, Cd, Ni: 0.1 ppm)

- Larutan Baku Multi-element 0,3 ppm/Multi-element standard solution 0.3 ppm

- Pipet 1,5 ml Larutan Baku Multi-element Induk; masukkan ke dalam labu ukur 50 ml.

Pipette 1.5 mL of Multi-element Standard Solution into a volumetric flask with a 50 mL capacity.

- Tambahkan larutan Asam Nitrat 2,5% hingga tanda batas volume, kocok hingga homogen.

To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Saring menggunakan syringe filter 0,2 µm.

Filter using a 0.2 µm syringe filter.

(Konsentrasi Pb, Cd, Ni: 0,3 ppm)/(Concentration of Pb, Cd, Ni: 0.5 ppm)

- Larutan Baku Multi-element 0,5 ppm/Multi-element standard solution 0.5 ppm

- Pipet 2,5 ml Larutan Baku Multi-element Induk; masukkan ke dalam labu ukur 50 ml.

Pipette 2.5 mL of Multi-element Standard Solution into a volumetric flask with a 50 mL capacity.

- Tambahkan larutan Asam Nitrat 2,5% hingga tanda batas volume; kocok hingga homogen.

To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Saring menggunakan syringe filter 0,2 µm.

Filter using a 0.2 µm syringe filter.

(Konsentrasi Pb, Cd, Ni: 0,5 ppm)/(Concentration of Pb, Cd, Ni: 0.5 ppm)

- Larutan Uji/sample solution

- Timbang 2 g zat uji, masukkan ke dalam labu ukur 20 ml.

Weigh 2 g of the test substance, put it in a 20 mL volumetric flask.

- Tambahkan larutan Asam Nitrat 2,5%; sonikasi hingga larut.

Add 2.5% nitric acid solution, sonicate until dissolved.

- Tambahkan larutan Asam Nitrat 2,5% hingga tanda batas volume; kocok hingga homogen.

To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Endapkan hingga endapan dan filtrat terpisah.

Precipitate until the filtrate and precipitate are separated.

- Saring filtrat menggunakan syringe filter 0,2 µm.

Filter the filtrate using a 0.2 µm syringe filter.

- Pengukuran/Measurement

Lakukan pemeriksaan sesuai dengan Instruksi Kerja Pengoperasian Alat Inductively Coupled Plasma.

Follow the Inductively Coupled Plasma Equipment Work Instructions for inspecting.

  - Kandungan Relatif Asam Stearat dan Asam Palmitat

Relative Content of Stearic Acid and Palmitic Acid

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

      - Larutan Boron Triflorida/Boron Trifluoride solution

- Timbang 14 g Boron Triflorida, masukkan ke dalam labu ukur 100 ml.

Weigh 14 g of Boron Triflouride, put into 100 mL volumetric flask.

- Larutkan dan encerkan sampai tanda batas menggunakan Metanol, kocok hingga homogen.

Dissolve and dilute to volume using Methanol, shake until homogenous.

      - Larutan Uji Kesesuaian Sistem/System Suitability solution

- Timbang 50 mg Asam Stearat dan 50 mg Asam Palmitat, masukkan ke dalam labu Erlenmeyer kecil yang disambungkan dengan kondensor refluks.

Weigh 50 mg Stearic acid and 50 mg Palmitatic acid, put into small Erlenmeyer flask connected into reflux condenser.

- Tambahkan 5 ml Larutan Boron Triflorida, refluks selama 10 menit sampai seluruh padatan terlarut.

Add 5 mL Boron Trifluoride solution, reflux for 10 minutes until the solids have dissolved.

- Tambahkan 4 ml n-Heptan (untuk kromatografi) melewati kondensor, refluks kembali selama 10 menit.

Add 4 mL of n-Heptane for chromatography through the condenser, and reflux again for 10 minutes.

- Dinginkan, lalu tambahkan 20 ml larutan Natrium Hidroksida jenuh, kocok, dan biarkan lapisan memisah.

Cool, then add 20 mL of saturated Sodium Hydroxide, shake, and allow the layers to separate.

- Alirkan lapisan n-Heptan melalui 0,1 g Natrium Sulfat anhidrat (yang sebelumnya dicuci dengan n-Heptan untuk kromatografi), tampung dalam wadah yang sesuai.

Flow the layers of n-Heptane through 0.1 g of Anhydrous Sodium Sulfate (previously washed with n-Heptane for chromatography), collect to suitable vial.

- Pipet 1 ml larutan, masukkan ke dalam labu ukur 10 ml.

Pipette 1 mL of solution, put into 10 mL volumetric flask.

- Encerkan sampai tanda batas menggunakan n-Heptan (untuk kromatografi).

Dilute to volume using n-Heptane (for chromatography).

      - Larutan Uji/Sample solution

- Timbang 100 mg zat uji, masukkan ke dalam labu Erlenmeyer kecil yang disambungkan dengan kondensor refluks.

Weigh 100 mg sample, put into small Erlenmeyer flask connected into reflux condenser.

- Tambahkan 5 ml Larutan Boron Triflorida, refluks selama 10 menit sampai seluruh padatan terlarut.

Add 5 mL Boron Trifluoride solution, reflux for 10 minutes until the solids have dissolved.

- Tambahkan 4 ml n-Heptan (untuk kromatografi) melewati kondensor, refluks kembali selama 10 menit.

Add 4 mL of n-Heptane for chromatography through the condenser, and reflux again for 10 minutes.

- DInginkan, lalu tambahkan 20 ml larutan Natrium Hidroksida jenuh, kocok, dan biarkan lapisan memisah.

Cool, then add 20 mL of saturated Sodium Hydroxide, shake, and allow the layers to separate.

- Alirkan lapisan n-Heptan melalui 0,1 g Natrium Sulfat anhidrat (yang sebelumnya dicuci dengan n-Heptan untuk kromatografi), tampung dalam wadah yang sesuai.

Flow the layers of n-Heptane through 0.1 g of Anhydrous Sodium Sulfate (previously washed with n-Heptane for chromatography), collect to suitable vial.

- Pipet 1 ml larutan, masukkan ke dalam labu ukur 10 ml.

Pipette 1 mL of solution, put into 10 mL volumetric flask.

- Encerkan sampai tanda batas menggunakan n-Heptan (untuk kromatografi).

Dilute to volume using n-Heptane (for chromatography).

      - Sistem Kromatografi/Chromatography system

Kolom/Column			: G16; 0,32 mm x 30 m; 0,5 µm

Suhu injector/Injector temp.	: 220°C

Suhu detektor/Detector temp.	: 260°C

Suhu kolom/Column temp.		:

| Temperatur Awal Initial Temperature (°) | Kenaikan Suhu Temperature Ramp (°/min) | Temperature Akhir Final Temperature (°) | Waktu Tahan pada Suhu Akhir Hold Time at Final Temperature (min) |
| --- | --- | --- | --- |
| 70 | - | 70 | 2 |
| 70 | 5 | 240 | 5 |

Detektor/Detector			: Flame ionization

Gas pembawa/Carrier gas		: Helium

Laju alir				: 2,4 ml/menit

Flow rate				: 2,4 mL/min

Volume injeksi/Injection volume	: 1 µl

      - Syarat Uji Kesesuaian Sistem/System suitability requirement

| Parameter | Kriteria Penerimaan/ Acceptance criteria |
| --- | --- |
| Resolusi/ Resolution | ≥ 5,0% antara Metil Palmitat dan Metil Stearat ≥ 5.0% between Methyl Palmitate and Methyl Stearate |
| Luas Area/Area | %RSD puncak larutan kesesuaian sistem ≤ 3,0% %RSD peak of system suitability solution ≤ 3.0% |
| Rasio Area/ Area ratio | %RSD puncak larutan kesesuaian sistem ≤ 1,0% %RSD peak of system suitability solution ≤ 1.0% |
| Waktu Retensi/ Retention time | 0,9 – 1,0 antara Metil Palmitat dan Metil Stearat 0.9 – 1.0 between Methyl Palmitate and Methyl Stearate |
| Faktor Ikutan/ Tailing Factor | Faktor ikutan puncak larutan standar : ≤ 2,0 Tailing factor peak of standard solutions : ≤ 2.0 |
| Total Plate Count | Total Plate Count : ≥ 2000 |

    - Perhitungan/Calculation :

C=LiLtx100

Keterangan/Note :

| C | = Kadar kandungan relatif Asam Stearat/Asam Palmitat (%) Relative content assay of Stearic acid/Palmitic acid (%) |
| --- | --- |
| Li | = Luas area Metil Stearat/Metil Palmitat pada Larutan uji Area Methyl Stearate/Methyl Palmitate of sample solution |
| Lt | = Total luas area Asam lemak pada Larutan uji Area total Fatty acid of sample solution |

  - Uji Batas Mikroba (R)/Microbial Limit Test (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Magnesium Stearate (S-QC-01-071).

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Cara Kerja/Procedure :

Lakukan pemeriksaan sesuai dengan Instruksi Kerja Pemeriksaan Uji Batas Mikroba (PQC-04-003).

Follow the Microbial Limit Test  Instruction for instruction (PQC-04-003).

- Daftar Lampiran/ List of Attachment

| Lampiran (Attachments) | No. Dokumen (Document No.) | Judul Dokumen (Document Title) | Tanggal Efektif (Effective Date) |
| --- | --- | --- | --- |
| - | - | - | - |

- Sejarah Revisi dan Review/ Revision and History Review

| Kode Dokumen (Document Code) | Revisi (Revision) | Tanggal Efektif (Effective Date) | Perubahan (Changes) | Alasan (Reasons) |
| --- | --- | --- | --- | --- |
| I-QC-01-015 | 00 | 27 SEP 2018 | - | Dokumen Baru New Document |
| I-QC-01-015 | 01 | 15 MAR 2022 | - Perubahan templet; Poin 5; Poin 7 Template change; point 3; point 5; point 7 | Review dokumen mengikuti kompendia dan aturan terbaru Document review following the latest kompendia and rules |
| I-QC-01-015 | 02 | 01 SEP 2022 | - Perubahan poin 5; Poin 6; Poin 7 Changes to 5 points; Point 6; 7 points | Review dokumen mengikuti referensi terbaru (USP 44, NF 39, 2021) Document review following the latest references (USP 44, NF 39; 2021) |
| I-QC-01-015 | 03 |  | - Penambahan parameter identifikasi B, Kandungan Asam Stearat dan Asam Palmitat dan Asam Palimitat Add parameter identification B, Relative content of Stearic Acid and Palmitic Acid - Penambahan alat dan bahan Add tool and material | Review dokumen mengikuti referensi terbaru Document review following the latest references |

- Daftar Distribusi Dokumen/Distribution List of Document

Asli/Original : SSDR

Controlled Copy:

      - AMPM

      - SPBB