- Tujuan/Objective

Untuk memastikan tata cara pemeriksaan bahan baku Croscarmellose Sodium dilakukan dengan baik dan benar sesuai spesifikasi yang berlaku.

To guarantee that the method for inspecting Croscarmellose Sodium raw materials is followed appropriately and in accordance with applicable spesifications.

- Ruang Lingkup/Scope

Laboratorium PT. Kimia Farma, Tbk - Plant Banjaran.

PT Kimia Farma Tbk. Plant Banjaran Laboratory.

- Penanggungjawab/Responsibility

Analis Pengawasan Mutu bertanggung jawab melaksanakan pemeriksaan bahan baku Croscarmellose Sodium sesuai dengan instruksi kerja ini.

The Quality Control Analyst is responsible for inspecting Croscarmellose Sodium raw materials in line with the working instructions.

- Definisi/Definition

-

- Referensi/Reference

USP 2022

- Alat dan Bahan/Equipments and Materials

  - Alat/Equipments :

| 1. | Timbangan Analitik/Analytical Balance | 14. | Cawan Pijar/Crucible |
| --- | --- | --- | --- |
| 2. | Gelas Kimia/Beaker Glass | 15. | Gelas Ukur/Measuring Cylinder |
| 3. | Tabung Reaksi/Test Tube | 16. | Corong Buchner/Buchner Funnel |
| 4. | pH Meter | 17. | Pipet Tetes/Drop Pipette |
| 5. | Oven | 18. | Erlenmeyer Berasah/Glass-Stoppered Conical Flask |
| 6. | Botol Timbang/Weighing Bottle |  | Buret/Burette |
| 7. | Desikator/Desiccator | 19. | Magnetic Stirrer |
| 8. | Furnace | 20. | Hot Plate |
| 9. | Pipet Ukur Measuring Pipette | 21. | Gelas Ukur Berasah Measuring Cylinder with Lid |
| 10. | Kertas Saring Filter Paper | 22. | Penjepit Cawan Crucible Tong |
| 11. | Autotitrator | 23. | Tangas air/Water bath |
| 12. | Spektrofotometer UV-Vis Spectrophotometer UV-Vis | 24. | Kawat Platina Platinum wire |
| 13. | Spektrofotometer Inframerah Spectrophotometer Infrared |  |  |

  - Bahan/Materials :

| 1. | Air Murni Purifed Water | 10. | Natrium Klorida (1 dalam 10) Sodium Chloride Solution (1 in 10) |
| --- | --- | --- | --- |
| 2. | Etanol Ethanol | 11. | Natrium Hidroksida 0,1 N LV 0.1 N Natrium Hidroksida Volumetric Standard |
| 3. | Biru Metilen (1 : 250.000) Methylene Blue (1 : 250,000) | 12. | Asam Klorida 0,1 N LV 0.1 N Hydrochloric Acid Volumetric Standard |
| 4. | m-Ungu Kresol LP m-violet cresol TS | 13. | Asam Sulfat P Sulfuric acid |
| 5. | 1-Naftol 1-Naphtol | 14. | 2,7-Dihidroksinaftalen 2,7-Dihydroxynaphthalene |
| 6. | Perak Nitrat 0,1 N 0,1 N Silver nitrate | 15. | Asam Glikolat Glycolate acid |
| 7. | Asam Asetat Glasial Glacial Acetic Acid | 16. | Aseton Acetone |
| 8. | Kalium Bromida P Potassium Bromide P | 17. | Kalium Karbonat 15% 15% Potassium Carbonate |
| 9. | Kalium Piroantimonat LP Potassium Pyroantimonate TS |  |  |

- Instruksi Kerja/Work Instruction

  - Pemerian (R)/Description (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

Amati secara visual (Organoleptis).

Observe visually (Organoleptic).

  - Kelarutan/Solubility

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

- Kelarutan dalam Air/Solubility in Water

- Larutkan 1 gram zat uji dengan 30  - 100 ml Air Murni dalam gelas kimia.

Dissolve 1 gram of the sample substance with 30 - 100 mL of Purified Water in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

- Kelarutan dalam Etanol/Solubility in Ethanol

- Larutan 10 mg zat uji dalam 100 ml Etanol dalam gelas kimia.

Dissolve 10 mg of the sample substance with 100 mL of Ethanol in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

- Kelarutan dalam Eter/Solubility in Ether

- Larutan 10 mg zat uji dalam 100 ml Eter dalam gelas kimia.

Dissolve 10 mg of the sample substance with 100 mL of Ether in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

- Kelarutan dalam Metanol/Solubility in Methanol

- Larutan 10 mg zat uji dalam 100 ml Metanol dalam gelas kimia.

Dissolve 10 mg of the sample substance with 100 mL of Methanol in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

- Kelarutan dalam Aseton/Solubility in Aceton

- Larutan 10 mg zat uji dalam 100 ml Aseton dalam gelas kimia.

Dissolve 10 mg of the sample substance with 100 mL of Aceton in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

- Kelarutan dalam Kloroform /Solubility in Chloroform

- Larutan 10 mg zat uji dalam 100 ml Kloroform dalam gelas kimia.

Dissolve 10 mg of the sample substance with 100 mL of Chloroform in a beaker glass.

- Amati pemerian larutan.

Observe the description of the solution.

  - Identifikasi (R)/Identification (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

  - Uji Identifikasi Inframerah/Infrared identification test

- Zat Baku/Standard substance

- Campur dan gerus 8 mg baku kerja Crosscarmellose Sodium dengan 400 mg Kalium Bromida kering.

Mix and grind 8 mg of Crosscarmellose Sodium working standard with 400 mg of dry Potassium Bromide in a mortar and pestle.

- Buat pellet Kalium Bromida.

Make Potassium Bromide pellets.

- Zat Uji/Sample substance

- Campur dan gerus 8 mg zat uji dengan 400 mg Kalium Bromida kering.

Mix and grind 8 mg of sample with 400 mg of dry Potassium Bromide in a mortar and pestle.

- Buat pellet Kalium Bromida.

Make Potassium Bromide pellets.

- Pengukuran/Measurement

- Lakukan “scanning” menggunakan Spektrofotometer IR pada panjang gelombang 400 - 4000 nm.

Use an IR spectrophotometer to do "scanning" at a wavelength of 400 - 4000 nm.

- Bandingkan spektrum zat uji dengan zat baku.

Compare the spectra of the test substance with the standard substance.

- Amati kedua spektrum tersebut, spektrum serapan inframerah zat uji menunjukkan maksimum hanya pada panjang gelombang yang sama seperti pada zat baku.

Observe the two spectra, the infrared absorption spectrum of the test substance shows a maximum only at the same wavelength as the standard substance.

  - Uji Identifikasi B/Identification test B

## Campurkan 1 g zat uji dengan 100 ml Larutan Biru Metilen (1 dalam 250.000).

## Mix 1 g of sample with 100 mL of Methylene Blue Solution (1 in 250,000).

## Aduk campuran dan biarkan mengendap.

## Stir the mixture and allow it  to settle.

## Amati secara visual.

## Observe visually.

  - Uji Identifikasi C/Identification test C

- Larutkan residu dari parameter Sisa pemijaran menggunakan 2 ml Air murni, ke dalam tabung reaksi.

Dissolve a residue from the Residue on ignition test using 2 mL Purified water.

- Tambahkan 2 ml Kalium Karbonat 15%, panaskan hingga mendidih : Tidak ada endapan muncul.

Add 2 mL of 15% Potassium Carbonate, heat to boiling : No precipitate is formed.

- Tambahkan 4 ml Kalium Piroantimonat LP, panaskan hingga mendidih.

Add 4 mL of Potassium Pyroantimonate TS, heat to boiling.

- Dinginkan di air es, apabila diperlukan gosok bagian dalam tabung reaksi dengan batang pengaduk : Muncul endapan pekat.

Cool using ice water, if necessary rub the inside of the test tube with glass rod : A dense precipitate is formed.

  - Sisa Pemijaran/Residue of ignition

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

- Pijarkan cawan pijar pada suhu 600 ± 50oC di dalam Furnace, selama 30 menit.

Incandescent crucible at a temperature of 600 ± 50oC in the Furnace, for 30 minutes.

- Dinginkan cawan pijar dalam desikator dan timbang (Wo).

Cool the crucible in a desiccator before weighing it (Wo).

- Timbang 1 g zat uji, masukkan ke dalam cawan pijar (Wa).

Weigh 1 g of sample, put into crucible (Wa).

- Tambah 1 ml Asam Sulfat P.

Add 1 mL of concentrated sulfuric acid.

- Panaskan sampai zat uji mengarang sempurna, lalu dinginkan.

Heat until sample substance is carbonized perfectly, then allow to cool.

- Basahi residu dengan 1 ml Asam Sulfat P.

Moisten the residue using 1 mL of concentrated Sulfuric Acid.

- Panaskan lagi hingga uap putih hilang.

Reheat until the white steam has dissipated.

- Pijarkan dalam Furnace pada suhu 600 ± 50oC hingga arang habis terbakar.

Incandescent inside furnace at 600 ± 50oC until the charcoal burns out.

- Dinginkan dalam desikator, timbang (Wb).

Cool in  desiccator, then weigh (Wb).

- Jika jumlah residu yang diperoleh melebihi batas yang ditetapkan, basahi lagi residu dengan 1 ml Asam Sulfat P.

If the amount of residue obtained exceeds the specified limit, moisten the residue again with 1 mL of Sulfuric Acid P.

- Panaskan lagi hingga uap putih hilang.

Reheat until the white steam is lost.

- Pijarkan dalam Furnace pada suhu 600 ± 50oC selama 30 menit.

Incandescent in the Furnace at a temperature of 600 ± 50 oC for 30 minutes.

- Dinginkan dalam desikator, timbang (Wb).

Allow to cool in desiccator, then weigh (Wb).

    - Perhitungan/Calculation :

SPn= Wb-WoWa-Wo ×100%(100%-SP) x 100%

Keterangan/Note :

SPn	= 	Sisa pemijaran (%)

Residue of ignition (%)

Wa	= 	Berat cawan pijar + zat uji sebelum pemijaran (gram)

Weight of crucible + test substance before ignition (grams)

Wb	= 	Berat cawan pijar + zat uji setelah pemijaran (gram)

Weight of crucible + sample substance after ignition (grams)

Wo	= 	Berat cawan pijar kosong (gram)

Weight of empty crucible (grams)

SP    =  Susut Pengeringan (%)

Loss on Drying (%)

  - Natrium Klorida/Sodium chloride

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

      - Timbang 5 g zat uji, masukkan ke dalam Erlenmeyer 250 ml.

Weigh 5 g of sample, put it into a 250 ml Erlenmeyer.

      - Tambah 50 ml Air murni dan 5 ml Hidrogen Peroksida 30%, panaskan di atas water bath selama 20 menit, aduk secara berkala.

Add 50 mL of Purified water and 5 mL of Hydrogen Peroxide 30%, and heat on a water bath for 20 minutes, stirring occasionally.

      - Dinginkan, lalu tambah 100 ml Air murni dan 10 ml Asam Nitrat.

Cool, and add 100 mL of Purified water and 10 mL of Nitric acid.

      - Titrasi dengan Perak Nitrat 0,05 N, tentukan secara potensiometri.

Titrate with 0.05 N Silver Nitrate, determining the endpoint potentiometrically.’

    - Perhitungan/Calculation :

C=584,4 x V x N100-SPx Wu

Keterangan/Note :

| C = | Kadar Natrium Klorida (%) Sodium chloride assay (%) |
| --- | --- |
| 584,4= | Faktor ekivalensi Natrium Klorida Equivalence factor for Sodium Chloride |
| V = | Volume Perak Nitrat 0,1 N (ml) Volume of 0,1 N Silver Nitrate (mL) |
| SP = | Susut pengeringan (%) Loss on drying (%) |
| Wu = | Berat zat uji yang ditimbang (mg) Weight of sample being weighed (mg) |

  - Natrium Glycolate/Sodium glycolate

    - Syarat/Requirement

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure

      - Larutan 2,7 – Dihydroxynaphtalene / 2,7 Dihydroxynaphtalene Solution

- Timbang 100 g 2,7-Dihydroxynapthalene, masukkan ke dalam labu ukur 1000 ml.

Weigh 100 g 2,7-Dihydroxynapthalene, put into a 1000 ml volumetric flask.

- Larutkan dan encerkan dengan Asam Sulfat P, kocok hingga homogen. Dan diamkan larutan hingga warna kekuningan menghilang.

Dissolve and dilute with Sulfuric Acid, shake until homogeneous. And let stand the solution until the yellowish color disappears.

(Jika larutan sangat gelap, jangan digunakan dan buat larutan baru menggunakan Asam Sulfat dari suplier lain. Larutan ini stabil sekitar 1 bulan jika disimpan dalam botol coklat/gelap)

(If the solution is very dark, discard it and prepare a new solution from a different supply of sulfuric acid. This solution is stable for approximately 1 month if stored in a dark bottle.)

      - Larutan Stok Standar/Standard stock solution

- Timbang 100 mg Asam Glikolat (yang telah dikeringkan dalam desikator dengan suhu ruang selama semalam) ke dalam labu ukur 100 ml.

Weigh 100 mg of Glycolic acid (that previously dried in a desiccator at room temperature overnight) to a 100 mL volumetric flask.

- Larutkan dengan Air murni beberapa ml, ultrasonik selama 10 menit.

Dissolve with a few mL of Purified water, ultrasonic for 10 minutes.

- Tambahkan Air murni sampai dengan tanda batas, kocok hingga homogen.

Add Purified water to volume, shake until homogeneous.

- Larutan Standar A/Standard solution A

- Pipet 1 ml Larutan stok standar ke dalam labu ukur 100 ml.

Pipette 1 mL of Standard stock solution into a 100 mL volumetric flask.

- Tambahkan Air murni hingga 5 ml dan 5 ml Asam asetat glasial.

Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid.

- Encerkan dengan Aseton sampai dengan tanda batas, kocok hingga homogen.

Dilute with Acetone to volume, shake until homogeneous.

- Larutan Standar B/Standard solution B

- Pipet 2 ml Larutan stok standar ke dalam labu ukur 100 ml.

Pipette 2 mL of Standard stock solution into a 100 mL volumetric flask.

- Tambahkan Air murni hingga 5 ml dan 5 ml Asam asetat glasial.

Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid.

- Encerkan dengan Aseton sampai dengan tanda batas, kocok hingga homogen.

Dilute with Acetone to volume, shake until homogeneous.

- Larutan Standar C/Standard solution C

- Pipet 3 ml Larutan stok standar ke dalam labu ukur 100 ml.

Pipette 3 mL of Standard stock solution into a 100 mL volumetric flask.

- Tambahkan Air murni hingga 5 ml dan 5 ml Asam asetat glasial.

Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid.

- Encerkan dengan Aseton sampai dengan tanda batas, kocok hingga homogen.

Dilute with Acetone to volume, shake until homogeneous.

- Larutan Standar D/Standard solution D

- Pipet 4 ml Larutan stok standar ke dalam labu ukur 100 ml.

Pipette 4 mL of Standard stock solution into a 100 mL volumetric flask.

- Tambahkan Air murni hinggal 5 ml dan 5 ml Asam asetat glasial.

Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid.

- Encerkan dengan Aseton sampai dengan tanda batas, kocok hingga homogen.

Dilute with Acetone to volume, shake until homogeneous.

- Larutan Uji/Sample solution

      - Timbang 500 mg zat uji, masukkan ke dalam gelas kimia 100 ml.

Weigh 500 mg of sample, put into 100 mL volumetric flask.

      - Basahi dengan 5 ml Asam asetat glasial dan 5 ml Air murni. Aduk menggunakan batang pengaduk selama 15 menit.

Wet thoroughly with 5 mL of Glacial acetic acid and 5 mL of Purified water. Stir with glass rod for 15 minutes.

      - Tambahkan 50 ml Aseton secara perlahan sambil diaduk.

Slowly add 50 mL of Acetone while stirring.

      - Tambahkan 1 g Natrium klorida, dan aduk beberapa menit (untuk memastikan pengendapan Karboksimetilselulosa yang sempurna).

Add 1 g of Sodium chloride, and stir for several minutes (to ensure complete precipitation of the Carboxymethylcellulose).

      - Saring menggunakan kertas saring (lembut dan berpori besar) yang sebelumnya telah dibasahi oleh Aseton, tampung dalam labu ukur 100 ml.

Filter using filter paper (soft and open-textured) that previously wetted with Acetone, collect the filtrate in a 100 mL volumetric flask.

      - Cuci padatan menggunakan 30 ml Aseton, kemudian tambahkan Aseton sampai tanda batas, kocok hingga homogen.

Wash the precipitate using 30 ml of Acetone, then add Acetone to volume, shake until homogeneous.

- Analisa/Analysis

      - Pipet 2 ml Larutan uji dan 2 ml masing-masing Larutan standar ke dalam labu ukur 25 ml.

(Gunakan 2 ml campuran 5% Asam asetat glasial dan 5% Air murni dalam Aseton sebagai blanko).

Pipette 2 mL of Sample solution and 2 mL of each Standard Solution into 25 mL volumetric flask.

(Use 2 mL of mixture 5% of Glacial acetic acid and 5% of Purified water in Acetone as blank).

      - Panaskan di atas water bath (labu ukur tanpa ditutup) selama 20 menit untuk menghilangkan aseton, lalu dinginkan.

Heat over a water bath (with uncovered flask) for 20 minutes to remove the acetone, then cool.

      - Tambahkan 5 ml 2,7-dihydroxynapthalene, aduk. Tambahkan 15 ml 2,7-dihydroxynapthalene, dan aduk kembali.

Add 5 mL of 2,7-dihydroxynapthalene, mix. Add 15 mL of 2,7-dihydroxynapthalene, and again mix.

      - Tutup mulut labu dengan aluminium foil, letakkan di atas water bath selama 20 menit.

Cover the mouth of flask with aluminum foil, place the flask over water bath for 20 minutes.

      - Dinginkan, kemudian tambahkan Asam sulfat hingga tanda batas, kocok hingga homogen.

Cool, then add Sulfuric acid to volume, shake until homogenous.

- Pengukuran/Measurement

Ukur absorbansi larutan menggunakan Spektrofotometer UV-Vis dengan panjang gelombang 540 nm.

Determine the absorbance of solution at 540 nm using Spectrofotometer UV-Vis at 540 nm.

    - Perhitungan/Calculate :

C=12,9 x Ws100- SPx Wu

Keterangan/Note :

| C = | Kadar Natrium Glikolat (%) Sodium Glycolate assay (%) |
| --- | --- |
| 12,9 = | Faktor konversi Asam Glikolat terhadap Natrium Glycolate Factor converting Glycolic acid to Sodium Glycolate |
| Ws = | Berat standar yang ditimbang ditentukan dari kurva standar dan absorbansi larutan sampel (mg) Weight of standard being weighed is determined from the standard curve and the absorbance of the sample solution (mg) |
| SP = | Susut pengeringan (%) Loss on drying (%) |
| Wu = | Berat zat uji yang ditimbang (mg) Weight of sample being weighed (mg) |

  - Bahan yang Larut dalam Air/Content of Water-Soluble Material

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

      - Timbang 10 g zat uji (W2), masukkan ke dalam gelas kimia 1000 ml.

Weight 10 g of sample (W2), put into 1000 mL of beaker glass.

      - Dispersikan dalam 800 ml Air Murni.

Dispersed in 800 mL of purified water.

      - Pada 30 menit pertama, setiap 10 menit larutan diaduk menggunakan magnetic stirrer selama 1 menit.

In the first 30 minutes, every 10 minutes the solution was stirred using a magnetic stirrer for 1 minute.

      - Biarkan selama satu jam lagi, atau sentrifus, jika perlu.

Allow to stand for an additional hour, or centrifuge, if necessary.

      - Endap-tuangkan 200 ml larutan ke corong buchner yang dilapisi kertas saring.

Decant 200 mL of the aqueous slurry into a buchner funnel lined with filter paper.

      - Masukkan 150 ml filtrat ke dalam gelas kimia 250 ml yang telah dipanaskan pada suhu 105oC, didinginkan, dan ditimbang (W3).

Put 150 mL of the filtrate into a 250 mL beaker glass that has been heated, cooled, and weighed (W3).

      - Kisatkan filtrat di atas hot plate hingga volume sedikit mungkin tapi tidak sampai kering.

Concentrate on a hot plate to a small volume, but not to dryness.

      - Keringkan pada suhu 105oC selama 4 jam.

Dry at 105oC for 4 hours.

      - Dinginkan ke dalam desikator selama 5 – 15 menit, timbang berat residu (W1).

Allow to cool in a desiccator for 5 – 15 minutes, weigh the residue (W1).

    - Perhitungan/Calculation :

W1= Wb-Wo

W3= Wf-Wo

BLA= [100 × W1 × 800+ W2]W2 × W3 × [1-0,01 × b]

Keterangan/Note :

BLA	= 	Bahan yang Larut dalam Air (%)

Content of Water-Soluble Material (%)

W1	= 	Berat residu (g)

Weight of the residue (g)

W2	= 	Berat zat uji yang ditimbang (g)

Weight of the sample substance being weighed (grams)

W3	= 	Berat filtrat (gram)

Weight of  the filtrate (grams)

b	= 	Susut Pengeringan (%)

Loss on drying (%)

Wo	=   Berat gelas kimia kosong (g)

Weigh of empty breaker glass (g)

Wb	=	Berat gelas kimia + residu (g)

Weight of breaker glass + residue (g)

Wf	=	Berat gelas kimia + filtrat (g)

Weight of breaker glass + filtrat (g)

  - Derajat Substitusi (R)/Degree of substitution (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

      - Timbang 1 g zat uji, masukkan ke dalam Erlenmeyer Berasah 500 ml.

Weigh 1 g of sample, put it in a  glass-stoppered 500 mL conical flask.

      - Tambahkan 300 ml larutan NaCl (1 dalam 10).

Add 300 mL of Sodium Chloride Solution (1 in 10).

      - Tambahkan melalui buret 25 ml NaOH 0,1 N LV.

From burette add 25 mL  of 0.1 N Sodium Hydroxide Volumetric Standard.

      - Tutup erlenmeyer dan biarkan selama 5 menit sambil dikocok sesekali.

Insert the stopper, and allow to stand for 5 minutes with intermittent shaking.

      - Tambahkan 5 tetes m-Ungu Kresol LP dan 15 ml HCl 0,1 N LV melalui buret, tutup Erlenmeyer berasah dan kocok.

Add 5 drops of m-violet cresol TS and  from a burette add 15 mL 0,1 N Hydrochloric Acid  Volumetric Standard; cover the glass-stoppered conical flask  and shake.

      - Jika larutan berwarna ungu, tambahkan HCl 0,1 N LV hingga larutan berwarna kuning (kocok pada setiap penambahannya).

If the solution is violet, add 0.1 N Hydrochloric Acid Volumetric Standard until the solution becomes yellow (shaking after each addition).

      - Titrasi dengan larutan NaOH 0,1 N LV hingga titik akhir berwarna ungu.

Titrate with 0.1 N Sodium Hydroxide Volumetric Standard to a violet endpoint.

    - Perhitungan/Calculated :

Miliekivalen basa yang digunakan untuk menetralkan 1 gram Croscarmellose Sodium dihitung terhadap berat kering :

Miliequivalents of base required for the neutralization of 1 g of Croscarmellose Sodium, on the dried basis :

M=V ×N ×100 %100% -SP

Derajat Substitusi Asam Carboxymethyl/The Degree of Acid Carboxymethyl Substitution :

A=1150 × M[7102-412 × M-80 × C]

Derajat Substitusi Natrium Carboxymethyl/The Degree of Sodium Carboxymethyl Substitution :

S=162+58 × A × C[7102-80 × C]

Derajat Substitusi/Degree of Substitution :

DS=A+S

Keterangan/Note :

V		=	Volume Titrasi (mL)

Titration Volume (mL)

N		=	Konsentrasi NaOH yang digunakan sebagai peniter (N)

Concentration of NaOH used as titrant (N)

M  	= 	Milliekivalen basa yang digunakan untuk menetralkan 1 gram zat uji dihitung terhadap berat kering (mek)

The milliequivalent of base used to neutralize 1 gram of sample calculated by dry weight (mek)

C		= 	Persentase Sisa Pemijaran (%)

Residue of ignition percentage (%)

A		=	Derajat substitusi Asam Carboxymethyl

Carboxymethyl acid degree of substitution

S		= 	Derajat substitusi Natrium Carboxymethyl

Sodium carboxymethyl degree of substitution

DS		= 	Derajat substitusi total

Total degree of substitution

  - Susut Pengeringan (R)/Loss on drying (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

- Panaskan botol timbang pada suhu 105oC di dalam Oven, selama 30 menit.

Heat the weighing bottle at a temperature of 105oC in the Oven, for 30 minutes.

- Dinginkan botol timbang dalam desikator dan timbang seksama (Wo).

Allow to cool the weighing bottle in a desiccator and weigh thoroughly (Wo).

- Bila zat uji berupa kristal besar, gerus secara cepat hingga ukuran partikel ± 2 mm.

If the test substance is big crystals, mill it quickly to a particle size of ± 2 mm.

- Timbang seksama 1 - 2 g zat uji, masukkan ke dalam botol timbang yang telah ditara (Wa).

Weigh carefully 1 - 2 g of the test substance, put it in a weighing bottle that has been weighed (Wa).

- Ratakan zat uji yang ada dalam botol timbang ± 5 mm – 10 mm.

Flatten the sample in the weighing bottle ± 5 mm – 10 mm.

- Masukkan ke dalam oven dengan suhu 105oC.

Put it in the oven at 105oC.

- Keringkan selama 6 jam.

Dry for 6 hours.

- Dinginkan dalam desikator selama 5 - 15 menit, kemudian timbang (Wb).

Allow to cool in a desiccator for 5 - 15 minutes , then weighing (Wb).

    - Perhitungan/Calculation :

SP= Wa-WbWa-Wo × 100%

Keterangan/Note :

SP		= 	Susut pengeringan (%)

Loss on drying (%)

Wa	= 	Berat botol timbang + zat uji sebelum pengeringan (gram)

Weight of weighing bottle + sample before drying (grams)

Wb	= 	Berat botol timbang + zat uji setelah pengeringan (gram)

Weight of weighing bottle + sample after drying (grams)

Wo	= 	Berat botol timbang kosong (gram)

Weight of empty weighing bottle (grams)

  - pH

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

      - Timbang 1 g zat uji, masukkan ke dalam gelas kimia.

Weigh 1 g of sample, put it in a beaker glass.

      - Tambahkan 100 ml Air Murni.

Add 100 mL of purified water.

      - Aduk selama 5 menit.

Stir for 5 minutes.

      - Lakukan pemeriksaan sesuai Instruksi Kerja Pemakaian Alat pH meter.

Follow the pH Meter Equipment Work Instructions when performing inspections.

  - Volume Pengendapan (R)/Settling Volume (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

      - Masukkan 75 ml Air Murni ke dalam gelas ukur berasah 100 ml.

Put 75 mL of purified water into 100 mL of measuring cylinder with lid.

      - Tambahkan 3 x 0,5 g zat uji, tutup dan kocok setiap kali penambahan.

Add 3 x 0.5 g of sample, close and shake every time adding.

      - Tambahkan Air Murni hingga 100 ml, kocok kembali hingga homogen.

Add purified water until 100 mL, shake until homogenous.

      - Biarkan selama 4 jam.

Let it for 4 hours.

      - Catat volume pengendapan.

Noted the volume of the settled mass.

  - Uji Batas Mikroba (R)/Microbial Limit Test (R)

    - Syarat/Requirement :

Sesuai dengan Spesifikasi Bahan Baku Croscarmellose Sodium (S-QC-01-128).

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Cara Kerja/Procedure :

Lakukan pemeriksaan sesuai prosedur pemeriksaan Uji Batas Mikroba (PQC-04-003).

Perform inspections according to the Microbial Limit Test (PQC-04-003) inspection procedure.

- Daftar Lampiran/ List of Attachment

| Lampiran (Attachments) | No. Dokumen (Document No.) | Judul Dokumen (Document Title) | Tanggal Efektif (Effective Date) |
| --- | --- | --- | --- |
| - | - | - | - |

- Sejarah Revisi dan Review/ Revision and History Review

| Kode Dokumen (Document Code) | Revisi (Revision) | Tanggal Efektif (Effective Date) | Perubahan (Changes) | Alasan (Reasons) |
| --- | --- | --- | --- | --- |
| I-QC-01-127 | 00 | 08 NOV 2018 | - | Dokumen Baru New Document |
| I-QC-01-127 | 01 | 01 SEP 2022 | - Perubahan templet dokumen; poin 1; poin 3; poin 5; poin 6; poin 7; Penerima distribusi Change of template document; point 1; point 3; point 5; point 6; point 7; Distribution recipient | Review dokumen mengikuti kompendia dan aturan terbaru (USP 44, NF 39, 2021) Document review following the latest kompendia and rules (USP 44, NF 39, 2021) |
| I-QC-01-127 | 02 | 17 MAR 2023 | Perubahan pada proses instruksi kerja Changes to the work instruction process | Review dokumen sesuai kompendia terbaru dan kebutuhan NIE Review documents according to the latest compendia and NIE needs |
| I-QC-01-127 | 03 |  | Perubahan Poin 6; Poin Identifikasi Changes to Point 6; Identification Point | Review dokumen sesuai kompendia terbaru dan kebutuhan NIE Review documents according to the latest compendia and NIE needs |

- Daftar Distribusi Dokumen/Distribution List of Document

Asli/Original : SSDR

Controlled Copy:

      - AMPM

      - SPBB