- Tujuan/Objective

Untuk memastikan tata cara pengawasan dalam proses pengemasan di Plant Banjaran dilakukan dengan baik dan benar.

To ensure that the packaging in process control procedure is carried out properly and correctly.

- Ruang Lingkup/Scope

Semua kegiatan yang berhubungan dengan pengawasan proses pengemasan di PT. Kimia Farma  Tbk Plant Banjaran.

All activities related to the packaging in process control of reference at PT. Kimia Farma Tbk  Plant Banjaran

- Penanggungjawab/Responsibility

  - Manager Pemastian Mutu menyetujui dan mengesahkan prosedur ini

Quality Assurance Manager approves and certifies this procedure

  - Asisten Manager Pemastian Operasional dan supervisor Inspektur Proses bertanggung jawab untuk menyiapkan, memeriksa dan memastikan prosedur ini dilaksanakan

Operational Assurance Assistant Manager and Process Inspecture Supervisor are responsible for preparing, checking and ensuring that these procedures are implemented

  - Pelaksana Operation Assurance bertanggung jawab untuk melaksanakan prosedur ini  dengan baik dan benar.

Operation Assurance staffs are responsible for carrying out this this procedure properly and correctly.

- Definisi/Definition

  - Pemeriksaan selama proses pengemasan merupakan aktifitas pemeriksaan pada tahapan-tahapan tertentu dalam proses pengemasan.

Inspection during the packaging process is an inspection activity at some stages in the packaging process.

  - Pengawasan adalah kegiatan untuk memastikan bahwa produk yang sedang diproses sesuai spesifikasi yang telah ditentukan.

Inspection is an activity to ensure that the product being processed conforms to determined specifications.

  - Bets adalah sejumlah produk obat yang mempunyai sifat dan mutu yang seragam yang dihasilkan dalam satu siklus pembuatan atas suatuperintah pembuatan tertentu

Batch is a number of the same character and quality medicinal product, produced in a single production cycle according ti specific manufacturing order

  - Nomor bets/No Lot Penandaan yang terdiri dari angka atau huruf atau gabungan keduanya, yang merupakan tanda pengenal suatu bets/lot , yang memungkinkan penelusuran kembali riwayat lengkap pembuatan bets/lot tersebut, termasuk tahap - tahap produksi, pengawasan dan distribusi.

Batch number /lot number is a code consisting of numbers or letters or a combination of the two , which is and identification of a batch/lot, which allows for full history of how the batch was made, including the stage of production, supervision, and distribution.

  - Tablet adalah sediaan padat mengandung bahan obat dengan atau tanpa bahan pengisi

Tablet is a solid unit dosage form each containing drug substances with or without excipient.

  - Serbuk adalah campuran kering bahan obat atau zat kimia yang dihaluskan, ditujukan untuk pemakaian oral atau untuk pemakaian luar

Powder is a mixtures of dry, finely divided drugs or chemical that may be intended for internal or external use

  - Kapsul adalah sediaan padat yang terdiri dari obat dalam cangkang keras atau lunak yang dapat larut

Capsule is a solid unit dosage form containing the drug which is enclosed in a hard or soft soluble container

  - Suspensi adalah sediaan cair yang mengandung partikel padat tidak larut yang terdispersidalam fase cair

Suspension is a liquid unit dosage form containing dispersion insoluble solid particle in liquid medium

  - Sirup adalah sediaan cair yang mengandung satu atau lebih zat kimia yang terlarut dalam pelarut atau campuran pelarut yang saling bercampur

Syrup is a liquid unit dosage form containing one or more chemical soluble in solvent or a blend of solvent

- Referensi/Reference

  - Pedoman Cara Pembuatan Obat yang Baik (CPOB) tahun 2018.

Guidelines for Good Medicine Manufacturing Practices (CPOB) 2018.

  - Pedoman Cara Pembuatan Obat Tradisional yang Baik Tahun 2020.

Guidelines for Making Good Traditional Medicines in 2020.

- Prosedur/Procedure

- Pemeriksaan Penandaan Bahan Kemas / The Inspection of Packaging Material coding

    - Periksa formulir Perintah Pembuatan untuk proses penandaan sesuai PGI dan Inspection Report ( Laporan Analisa )

Check the Manufacturing Order form for the coding process to comply with PGI and Inspection Report.

    - Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line clearance refers to Line Clearance Procedure (PQS-02-004). Record in PGI

    - Periksa identitas bahan kemas dan dokumen yang digunakan (PGI dan Laporan Analisa)

Check the identity of the packaging material and documents used (PGI and Analysis Report)

    - Periksa hasil set up penandaan strip atau bahan kemas lainnya yang sudah ditandai dan sudah diperiksa oleh Supervisor pengemasan meliputi :

Check the results of coding strips or other packaging materials that have been signed and checked by the packaging supervisor, including:

- No Bets / No. Batch

- Mfg date (jika ada / if any)

- Expired date.

- HET

- Estetika / Aesthetics

    - Bubuhkan paraf dan tanggal pada bahan kemas yang sudah sesuai penandaannya

Give a sign and date to packaging materials that have been properly coded

- Catat hasil pemeriksaan di logbook

Record the inspection report in the logbook.

- Pemeriksaan Selama Proses Pengemasan Strip/ Blister

In-Process of stripping /Blistering Inspection

    - Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

    - Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI, Laporan Analisa, Label Diluluskan, pemeriksaan massa cetak dan ruahan).

Check the identity of products, packaging materials, and documents used (PGI, Analysis Report, Released Product label, intermediate products inspection).

    - Ambil dan periksa hasil setting strip/ blister sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and check of the stripping / blistering result according to PGI, including:

- No. Bets.

No. Batch

- Mfg date (jika ada / If any)

- Exp date

- HET

- Jumlah tablet/ kapsul

The quantity of tablets / capsules

- Kondisi tablet/ kapsul

Condition of tablets / capsules

- Kondisi seal / Condition seal

- Estetika

Aesthetics

    - Periksa kebocoran dengan alat Eksikator Untuk Kebocoran (I-QC-00-070) atau Alat vacuum Leak Tester Erweka VDT/S (I-QC-00-151).

Check for leaks with an Exicator leaked tester (I-QC-00-070) or Erweka VDT/S leaked tester ((I-QC-00-151).

    - Catat hasil pemeriksaan pada Formulir In- Process Pengemasan Primer Strip/ Blister (F-PQS-02-010) pada set up (awal), tengah dan akhir proses pengemasan.

Record the inspection report in In-Process Inspection Stripping/ Blistering Form (F-PQS-02-010-02) at the set-up (the beginning), the middle, and the end  process.

- Catat hasil pemeriksaan di logbook

Record the inspection report in the logbook.

- Pemeriksaan Selama Proses Pengemasan Serbuk

In-Process of sacheting Inspection

      - Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line Clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

      - Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI, Laporan Analisa, Label Diluluskan)

Check the identity of products, packaging materials, and documents used (PGI, Analysis Report, Released Product Label).

      - Ambil dan periksa hasil setting sase sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and check of the filling and sacheting result according to PGI, including:

- No. Bets.

No. Batch

- Mfg date (jika ada/ if any)

- Exp date

- HET

- Berat (14 sase)

Weight (14 sachet)

- Kebocoran (7 sase)

Leak test (7 sachet )

- Kondisi seal/ Condition seal

- Estetika /Aesthetics

      - Catat pemeriksaan pada Formulir In-Process Control  Pengemasan Serbuk (F-PQS-02-010-05) pada set up (awal), tengah dan akhir proses sacheting.

Record the inspection results in In-Process Inspection sacheting Form (F-PQS-02-010-05). at the set-up (the beginning), the middle,  and the end process.

      - Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

      - Ambil 6 sase dan kirim ke QC.

Take 6 sachets and send them to QC.

- Pemeriksaan Selama Proses pengemasan primer sirup dan suspensi

In-Process of filling Inspection for Syrup and Suspension

- Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line Clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

- Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI, Laporan Analisa, Label Diluluskan)

Check the identity of products, packaging materials, and documents used (PGI, Analysis Report, Released Product Label)

- Ambil dan periksa hasil pengisian sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and check the filling bottle results according to PGI, including:

- Volume terpindahkan.

Volume

- Daya cengkram/ torsi.

Torque.

- Kebocoran.

Leak test

- Kejernihan/ clearness

- Estetika.

Aesthetics.

- Catat pada formulir pemeriksaan In Process Control Sirup/suspensi/Cairan Herba l(F-PQS-02-010-06) pada set up (awal), tengah dan akhir proses pengisian.

Record the inspection results in In-Process Inspection syrup/suspension/Traditional Medicine Liquid Form  (F-PQS-02-    010-06) at the set-up (the beginning), the middle, and the end process.

- Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

- Ambil 3 botol dan kirim ke QC

Take 3 bottles and send them to QC.

- Pemeriksaan Selama Proses Pengemasan Primer Cairan Herbal

In-Process of filling Inspection for Traditional Medicine Liquid

- Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line Clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

- Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI, Laporan Analisa, Label Diluluskan)

Check the identity of products, packaging materials, and documents used (PGI, Analysis Report, Released Product Label)

- Ambil dan periksa hasil pengisian sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and checks the filling results according to PGI, including:

- Volume terpindahkan.

volume

- Daya cengkram/ torsi.

Torque.

- Kebocoran.

Leak test

- Kejernihan/ clearness

- Estetika.

Aesthetics.

- Catat pada formulir pemeriksaan In Process Control sirup/suspensi/cairan Herbal  (F-PQS-02-010-06) pada set up (awal), tengah dan akhir proses pengisian.

Record the inspection results in In-Process Inspection syrup/suspension/Traditional Medicine Liquid Form  (F-PQS-02-    010-06) at the set-up (the beginning), the middle, and the end process.

- Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

- Ambil 1 botol dan kirim ke QC (lab. Mikrobiologi)

Take 1 bottle and send it to QC.

- Pemeriksaan Selama Proses Pengemasan Tablet kemasan botol

In- process of filling Inspection for tablets

      - Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line Clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

      - Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI, Laporan Analisa, Label Diluluskan)

Check the identity of products, packaging materials, and documents used (PGI, Analysis Report, Released Product Label)

      - Ambil 6 botol dan periksa hasil pengisian sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and checks 6 bottle of the filling results according to PGI, including:

- Jumlah / Count

      - Catat pada formulir pemeriksaan In Process Control sirup/suspensi/cairan Herbal/tablet/kapsul (F-PQS-02-010-06) pada set up (awal), tengah dan akhir proses pengisian.

Record the inspection results in In-Process Inspection syrup/suspension/Traditional Medicine Liquid/Tablets/Capsule Form  (F-PQS-02-    010-06) at the set-up (the beginning), the middle, and the end process.

      - Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

- Pemeriksaan Selama Proses Pengemasan Sekunder

In- process Secondary Packaging Inspection

  - Tanpa QR Code/ Without QR Code

      - Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line clearance refers to the Line Clearance Procedure (PQS- 02-004). Record in PGI.

      - Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI)

Check the identity of products, packaging materials and document used (PGI)

      - Ambil dan periksa produk sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and check the product according to PGI, including:

- Kondisi dus dan kesesuaiannya

Box condition and suitability

- No. Bets.

No. Batch

- Mfg date (jika ada / if any)

- Expire date

- HET

- Jumlah

Quantity

      - Catat pada formulir pemeriksaan In-Process Control Pengemasan Sekunder (F-PQS-02-010-07) pada set up (awal), tengah dan akhir proses pengemasan.

Record of the Secondary Packaging In-Process Control inspection form (F-PQS-02-010-07) at the set up (beginning), middle and end of the packaging process.

      - Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

      - Ambil sampel dan sampel pembanding sesuai Prosedur Pengambilan dan Penanganan Sampel Pembanding dan Sampel Pertinggal Produk Jadi (PQS-02-012).

Take samples and comparison samples in accordance with the Procedure for Taking and Handling References and retained Sample of Finished Goods.

  - Dengan QR Code / With QR Code

- Periksa Kesiapan jalur (line clearance) mengacu pada Prosedur Line Clearance (PQS-02-004). Catat di PGI

Check Line clearance refers to the Line Clearance Procedure (PQS-02-004). Record in PGI.

- Periksa identitas produk, bahan kemas dan dokumen yang digunakan (PGI)

Check the identity of products, packaging materials and document used (PGI).

- Ambil dan periksa produk sesuai Prosedur Pengemasan Induk (PGI) meliputi :

Take and check the product according to PGI including:

- Kondisi dus dan kesesuaiannya

Box condition and suitability

- No. Bets.

No. Batch

- Mfg date (jika ada /If any)

- Expire date

- HET

- Jumlah

Quantity

- QR Code

- Catat pada formulir pemeriksaan In Process Control Pengemasan Sekunder (F-PQS-02-010-07) pada set up (awal), tengah dan akhir proses pengemasan.

Record of the Secondary Packaging In-Process Inspection form (F-PQS-02-010-07) at the set up (the beginning), the middle and the end of the packaging process.

- Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

- Ambil sampel pertinggal dan sampel pembanding sesuai Prosedur Pengambilan dan Penanganan Sampel Pembanding dan Sampel Pertinggal Produk Jadi (PQS-02-012).

Take retain samples and comparison samples in accordance with the Procedure for Taking and Handling References and retained Sample of Finished Goods.

- Pemeriksaan Produk Jadi

Finished Product Inspection

      - Periksa produk jadi sesuai PGI, meliputi:

Check Finished Product according to PGI , including :

- Nama produk dan kekuatan sediaan.

Product name and preparation strength.

- No. Bets / No Batch

- Expired Date.

- Isi/ Content.

- Berat Box.

Weight

- Jumlah box.

Quantity

- QR Code (jika ada / if any)

      - Periksa data berat box hasil timbang yang dilakukan oleh produksi, periksa hasil timbangan terendah dan tertinggi.

Check the weight data of the weighing box results carried out by production, verify the lowest weighing and the highest weighing results.

      - Hitung jumlah box yang sudah ditimbang.

Count the quantity of boxes that have been weighed.

      - Ambil dan periksa sampel sesuai tabel pada Military Standard ditambah box yang tidak standar (termasuk box berat terendah dan berat tertinggi).

Take and check samples according to the table on Military Standards plus non-standard boxes (including the lowest-weight box and the highest-weight box).

      - Buka penutup box dan periksa isi box sesuai tabel Military Standard meliputi:

Open the box cover and check the contents according to the table on Military Standard, including :

  - Nama produk dan kekuatan sediaan.

Product name and dosage

  - Jumlah dus/ content

  - No. Bets / No Batch

  - Mfg Date.

  - Expire Date

  - Jumlah strip/blister/sase/botol

Strips/Blisters/sachet/bottle count

  - QR Code (jika ada/if any)

- Cap sampled  pada box, paraf dan tanggal pemeriksa.

Sampled stamp on the box, inspector’s initial and date.

- Tempel label : PRODUK TELAH DIPERIKSA IPC

Labeled by : “PRODUCT HAS BEEN INSPECTED BY IPC”.

- Catat pemeriksaan pada formulir Pemeriksaan Pengemasan Akhir Produk Jadi 
(F-PQS-02-010-04).

Record on the Finished Product Packaging Inspection form (F-PQS-02-010-04).

- Catat hasil pemeriksaan di logbook

Record the inspection results in the logbook.

- Daftar Lampiran/List of Attachment

| Attachments | Document No. | Document Title | Effective Date |
| --- | --- | --- | --- |
| 1 | F-PQS-02-010-01/02 | Lembar Cap SAMPLED SAMPLED Stamp Sheet |  |
| 2 | F-PQS-02-010-02/02 | Formulir Pemeriksaan Pengemasan Primer Strip/Blister Primary Strip/Blister Packaging Inspection Form |  |
| 4 | F-PQS-02-010-04/02 | Formulir Pemeriksaan Pengemasan Akhir Produk Jadi Finished Product Final Packaging Inspection Form |  |
| 5 | F-PQS-02-010-05/02 | Formulir In-Process Control Serbuk Powder In-Process Control Form |  |
| 6 | F-PQS-02-010-06/02 | Formulir In-Process Control Cairan Oral Oral Liquid In-Process Control Form |  |
| 7 | F-PQS-02-010-07/02 | Formulir In-Process Control Pengemasan Sekunder Secondary Packaging In-Process Control Form |  |

- Sejarah Revisi Prosedur/Procedure Revision History

| Document Code | Revision | Effective Date | Changes | Reasons |
| --- | --- | --- | --- | --- |
| PQS-02-010 | 00 | 09 Juli 2019 | - | Dokumen baru New document |
| PQS-02-010 | 01 |  | - Pada referensi terdapat Pedoman CPOB 2012 diganti menjadi CPOB 2018. In reference there is the CPOB 2012 changed to CPOB 2018. - Penghapusan Petunjuk Operasional CPOB 2012. Elimination of CPOB 2012 Operational Instructions. - Dihapuskan referensi PT. Kimia Farma (Persero) Tbk Plant Banjaran. Removed reference to PT. Kimia Farma Factory (Persero) Tbk Banjaran. - Pada poin 6. Alat/tools dihapus. At point 6. Tools removed. - Pada poin 6.5.2 tertulis 10 botol untuk retained sample diganti menjadi 5 botol untuk retained sample. In point 6.5.2 it says 10 bottles for retained samples changed to 5 bottles for retained samples. - Pada poin 6.6.1 ditambah poin g. leaflet. In point 6.6.1 plus point g. leaflet. - Pada poin 6.6.3 dan 6.7.1 ditambahkan poin leaflet. Points 6.6.3 and 6.7.1 added leaflets point. - Pada poin 8 lampiran, penomoran lampiran diurutkan. At point 8 attachments, attachment numbering is sorted. | Mengikuti CPOB 2018 dan penyesuaian dilapangan terkait prosedur, selain itu ada redaksi penulisan pada nomor lampiran. Following CPOB 2018 and adjustments in the field regarding procedures, apart from that there is a writing editorial in the attachment number. |
| PQS-02-010 | 02 |  | Perubahan Point 6.2; 6.3; 6.3.1; 6.3.2; 6.3.3; 6.3.4; 6.4; 6.5; 6.6; 6.7 Penambahan Point 6.8 Penghapusan lampiran 3 Penambahan Lampiran 5,6 dan 7. Changes to Point 6.2; 6.3; 6.3.1; 6.3.2; 6.3.3; 6.3.4; 6.4; 6.5; 6.6; 6.7 Point Addition 6.8 Attachment deletion 3 Addition of Attachments 5, 6, and 7. | Mengikuti kondisi di lapangan Following the conditions in the field |

- Review Dokumen/Document Review

| Review | Date | Review Output | Reviewer | Sign |
| --- | --- | --- | --- | --- |
| - | - | - | - | - |

- Daftar Distribusi Dokumen/Distribution List of Document

Asli: SQC

Controlled Copy:

| 1. | AMQA | 8. | SPP3 |
| --- | --- | --- | --- |
| 2. | AMP | 9. | SPPH |
| 3. | AMQC | 10. | SCRP |
| 4. | SQOA | 11. | SCPRP |
| 5. | SQTS | 12. | SCRPH |
| 6. | SPP1 | 13. | SCMW |
| 7. | SPP2 | 14. | SCSR - Periksa hasil set up dilakukan pada saat: Inspection of the Set-Up Process is carried out at the time: - Mulai Proses Produksi/ Pengemasan/ Penandaan Start the Production/Packaging/tagging process. - Setelah Perbaikan Mesin Produksi. After Production Machine Repair. - Tambahan Bahan Kemas Susulan. Addition ingredient pack continuation. - Pergantian Shift/ Hari. Change of Shift/Day. - Ganti Mesin Produksi/ Pengemasan. Replace Production/Packaging Machines. |