- Tujuan/Objective

Memastikan tata cara penanganan penyimpanan material dilakukan dengan baik dan benar sesuai prosedur.

Ensure that the procedures for handling material storage are carried out properly and correctly according to procedures.

- Ruang Lingkup/Scope

PT Kimia Farma Tbk, Plant Banjaran.

- Penanggungjawab/Responsibility

- Manager Pemastian Mutu bertanggung jawab mengkaji dan menyetujui prosedur tetap ini.

Quality Assurance Manager Assistant is responsible for reviewing and approving this procedure.

- Asisten Manager Penyimpanan bertanggung jawab memeriksa, dan memastikan prosedur tetap ini.

Warehouse Assistant Manager is responsible for checking and ensuring this procedure.

- Supervisor Gudang Bahan Baku bertanggung jawab menyiapkan, memeriksa secara berkala, dan memastikan prosedur ini dilaksanakan secara konsisten, baik dan benar.

Raw Material Warehouse Supervisor are responsible for preparing, reviewing periodically, and ensuring this procedure is consistently and properly implemented

- Definisi/Definition

  - Bahan baku adalah semua bahan yang berkhasiat maupun tidak berkhasiat, yang berubah maupun tidak berubah yang digunakan dalam pengelolahan obat maupun tidak semua bahan tersebut masih terdapat di dalam produk ruahan/jadi.

Raw materials are all materials that are efficacious or not efficacious, which change or do not change, which are used in drug processing or not, all of these materials are still contained in the bulk/finished product.

  - Eksipien (bahan tambahan) adalah suatu bahan, bukan berupa zat aktif yang telah dievaluasi dengan benar keamanannya dan termasuk dalam sistem penghantaran obat (drug delivery system) untuk :

Additive is a substance ,not an active substance that has been properly evaluated for safety and is included in the drug delivery system for :

    - Membantu dalam memproses sistem penghantar obat selama pembuatan obat tersebut

Assist in processing the drug delivery system during the manufacturing.

    - Melindungi, mendukung atau meningkatkan stabilitas obat, ketersediaan hayati (bioavaibility), atau akseptibilitas pasien

Protect, support or enhance drug stability, bioavailability or patient acceptability

    - Membantu identifikasi produk, atau/Assist in product identification ,or

    - Meningkatkan atribut lain yang berkaitan dengan keamanan dan efektifitas obat selama penyimpanan atau penggunaan.

Improve other attributes related to the safety and effectiveness of drugs  during storage or use.

  - Psikotropika memiliki potensi mengakibatkan sindroma ketergantungan terhadap obat jenis tertentu.

Psychotropic have the potential to cause a dependence syndrome on certain types of drugs.

  - Prekursor farmasi adalah zat atau bahan pemula atau bahan kimia yang dapat digunakan sebagai bahan baku/penolong untuk keperluan proses produksi industri farmasi atau produk antara, produk ruahan,dan produk jadi yang mengandung efedrin, pseudoefedrin, norefedrin/fenilpropanolamin, ergotamin, ergometrin, atau potassium permanganat.

Pharmaceutical precursors are subtances or starting materials or chemicals that can be used as raw/auxiliary materials for the purposes of the pharmaceutical industry production process or intermediate products, bulk products and finished products containing ephedrine,ergotamine,ergometrine or potassium permanganate.

  - Bahan kemas adalah tiap bahan, termasuk bahan kemas cetak yang digunakan dalam proses pengemasan produk jadi, tetapi tidak termasuk kemasan luar yang digunakan untuk transportasi atau keperluan pengiriman keluar pabrik. Bahan pengemas terdiri dari bahan pengemas primer (bersentuhan langsung dengan produk) dan sekunder.

Packaging material is any material including printed packaging materials used in the process of packaging of finished product, but excluding outer packaging used for transportation or shipping purposes outside the factory, packaging materials consist of primary packaging materials (direct contact with the product) and secondary packaging materials.

  - Yang dimaksud material pada prosedur ini adalah bahan baku, bahan kemas, produk jadi, produk antara, dan produk ruahan.

Materials in this procedure are  raw materials, finished product, intermediate product and bulk products.

- Referensi/Reference

  - Pedoman dan petunjuk operasional CPOB 2018

GMP 2018 operational guidelines and instructions

  - Pedoman dan petunjuk operasional CPOTB 2011

GMP 2011 operational guidelines and instructions

- Prosedur/Procedure

  - AREA MATERIAL WAREHOUSE +0.000

    - Bahan baku yang harus disimpan di suhu maksimum 25OC disimpan dalam ruangan raw material warehouse I & II +0.000 (kode ruangan 1511039 & 1511038).

Raw materials that must be stored at a maximum of 25 C are stored in the raw material warehouse 1 & 2 (room code 1511029 & 1511038)

    - Bahan baku empty capsul disimpan di suhu maksimum 25OC dan kelembaban 35-65%, disimpan dalam ruangan empty capsul  (kode ruangan 1511012).

Empty capsule raw materials are stored at a maximum temperature of 25OC and 35-65% humidity, stored in an empty capsule room (room code 1511012).

  - AREA PRIMARY PACKAGING WAREHOUSE +0.000

    - Bahan kemas primer disimpan dalam ruangan primary packaging warehouse I +0.000 (kode ruangan 1511040)

Primary packaging materials are stored in the primary packaging warehouse 1  +0.000 (room code 1511040)

  - AREA WEIGHING +0.000

    - Menerima Bahan baku yang dikirim dari warehouse menuju area deduster material dan airlock material berdasarkan prosedur dengan nomor prosedur PWR-03-006.

Receive raw materials from the warehouse to the deduster material area and material airlock according to the procedure PWR -03-006

    - Bahan baku yang menunggu proses penimbangan disimpan diruangan staging before weighing +0.000 (kode ruangan 1513013)

Raw materials waiting for the weighing process are stored in staging before weighing room (room code 1513013).

    - Setelah proses penimbangan selesai dan menunggu proses produksi selanjutnya, bahan baku disimpan diruangan staging after weighing +0.000 (kode ruangan 1513021).

After the weighing process is complete and waiting for the nex production process, the raw materials are stored in the after weighing staging room (room code 1513021)

    - Bahan baku sisa penimbangan disimpan dalam ruang retained raw material +0.000 (kode ruangan 1513012, 1513007, 1513013) dan cold storage weighing +0.000 (kode ruangan 1513011).

The remaining raw materials for weighing are stored in the retained raw material (room code 1513012) and cold storage weighing rooms (room code 151311)

  - AREA GRANULATION AND TABLETING NON BL +0.000

    - Produk antara/masa cetak hasil dari produksi granul di simpan dalam ruangan WIP intermediate non BL +0.000 (kode ruangan 1513044).

Intermediate products /tableting mass from granulation are  storage in a non-BL.wip intermediate, +0.000 ( room code 1513044 )

    - Produk ruahan dari proses granulasi disimpan diruangan WIP bulk non BL +0.000 (kode ruangan 1513060).

Bulk products from the granulation process stored in the WIP bulk non BL room +0.000 (room code 151311)

  - AREA PHARMA 1 LINE 4 +0.000

PHARMA AREA 1 LINE 4 +0.000

    - Bahan baku yang menunggu untuk ditimbang disimpan diruangan weighing TB +0.000 (kode ruangan 1513075).

Raw materials waiting to be weighed are stored in the weighing TB +0.000(room code 1513075 )

    - Setelah bahan baku selesai ditimbang, bahan baku yang menunggu untuk di proses produksi disimpan diruangan after weighing TB +0.000(kode ruangan 1513076).

After the raw materials have been weighed ,the raw materials for production process are stored in after weighing TB room +0.000 (room code 1513076 )

    - Bahan baku cangkang kapsul kosong yang akan menunggu untuk proses filling capsule disimpan diruangan empty capsule TB +0.000 (kode ruangan 1513088).

Empty capsule raw materials  that will wait for the capsule filling process are stored in the empty capsul room TB . +0.000( room code 1513088 )

    - Untuk masa cetak atau tablet yang akan di coating disimpan didalam ruangan WIP TB +0.000 (kode ruangan 1513099).

For Granules or tablets to be coated stored in the WIP TB room , +0.000 ( room code 1513099 )

  - AREA LIQUID +0.000

LIQUID AREA +0.000

    - Bahan baku untuk proses produksi liquid disimpan diruangan staging RM liquid +0.000 (kode ruangan 1513119).

Raw materials for the liquid production process are stored in the RM liquid staging room, +0.000 ( room code 1513119 )

    - Untuk bahan kemas capping botol disimpan diruangan staging primary packaging +0.000 (kode ruangan 1513124A).

Bottle capping packaging materials are stored in the primary packaging staging room, +0.000 ( room code 1513124A )

    - Bahan kemas botol kaca dan botol plastik dimasukkan ke dalam ruangan staging secondary packaging +0.000 (kode ruangan 153159).

Glass bottles and plastic bottles are put inside secondary packaging staging room ,+0.000 ( room code 153159 )

    - Bahan kemas botol plastik atau kaca yang dikarantina disimpan diruangan (kode ruangan 1514007)

Plastic or glass bottle warehouse  that are quarantined are stored in quarantine bottle area +0.000 (kode area 1514007).

    - Produk liquid yang telah dikemas kemudian disimpan dalam ruangan bottle warehouse +0.000 (kode ruangan 1514008).

Liquid product that have been packaged are then stored in the bottle warehouse ,+0.000 ( room code 1514008 )

  - AREA FINISHED GOODS WAREHOUSE PHARMA DAN HERBAL +0.000

    - Produk jadi Farma, Herbal, produk jadi psikotropik, precursor, dan OOT yang dikarantina, disimpan dalam area Quarantine Finished Goods +0.000 (kode area 1514011).

Pharma and Herbal Finished products and psychotropic and precursor finished products quarantined are stored in the finished good quarantine area (area code 1514011)

    - Produk jadi Pharma, Herbal, produk jadi psikotropik, precursor, dan OOT yang siap untuk dikirimkan disimpan dalam area release Finished Goods Warehouse +0.000 (kode ruangan 1514012).

Pharma and Herbal Finished products and psychotropic & precursor products that are ready to be shipped are stored in the released area finished good warehouse +0.000 (room code 1514012 )

    - Bahan Kemas Sekunder yang dikarantina disimpan dalam area  quarantine/racking terpisah di Finished Goods Warehouse +0.000 (kode ruangan 1514012).

Secondary Packaging Materials that are quarantined are stored in separated area/racking in the finished good warehouse +0.000 (room code 1514012 )

  - AREA FINISHED GOODS WAREHOUSE +4.800

- Penyimpanan Produk Pharma Kembalian/Ditolak dan produk jadi psikotropik & prekursor di simpan dalam ruang Reject Recall Finished Goods +4.800 (kode ruangan 1514020)

Storage of Returned/Rejected Pharma Products and finished psychotropic & precursor products stored in the Reject Recall Finished Goods room +4,800 (room code 1514020)

  - PHARMA AREA FINISHED GOODS WAREHOUSE +9.000

- Penyimpanan bahan kemas hasil penandaan di simpan dalam ruangan Staging After Coding +9.000.(kode area 1525008)

Storage of after coding packaging materials are stored in the staging room after coding +9.000 (area code 1525008)

- Area Coding 3 untuk penandaan dus dan etiket +9.000 (kode ruangan 1525008)

Coding Area 3 for box marking and label +9,000 (room code 1525008)

  - AREA RAW MATERIAL WAREHOUSE +12.000

    - Penyimpanan material bahan baku yang disimpan dalam suhu dingin antara 2O - 8OC disimpan dalam raw material cold storage 2O - 8OC +12.000 (kode ruangan 1521018).

Storage of raw materials that are stored in cold temperatur between 2-8 0C are storage in raw material cold storage 2 – 8 0C  +12000(room code 1521018)

    - Penyimpanan bahan baku disimpan dalam ruangan raw material warehouse +12.000 (kode ruangan 1521017).

Raw materials are stored in the raw material warehouse +12.000 ( room code 1521017 )

  - AREA PRIMARY PACKAGING WAREHOUSE +12.000

    - Bahan kemas primer di simpan dalan ruangan primary packaging warehouse +12.000 (kode ruangan 1521016).

Primary packaging Materials are stored in primary packaging warehouse +12.000 (room code 1521016)

  - AREA SECONDARY PACKAGING WAREHOUSE 2ND Floor +12.250

    - Bahan kemas sekunder disimpan didalam ruangan secondary packaging warehouse 2nd floor +12.250 (kode ruangan 1525003).

Secondary packaging material are stored in room secondary packaging warehouse 2 floor + 12.250 ( room code 1525003 )

    - Penyimpanan bahan kemas yang akan dilakukan penandaan/Before Coding di simpan dalam ruangan secondary packaging warehouse  2nd flor +12.250 (kode ruangan 1525003)

Storage of packaging materials that will be tagged/Before Coding is stored in the secondary packaging warehouse room 2nd flor +12,250 (room code 1525003)

    - Area Coding 1 untuk penandaan Box ( kode ruangan 1525005)

Coding Area 1 for marking Box (room code 1525005)

    - Area Coding 2 untuk Penandaan dus dan etiket ( kode ruangan 1525004)

Coding Area 2 for box marking and labeling (room code 1525004)

  - AREA QC +9.000

    - Material yang akan di trial disimpan didalam ruangan staging trial +9.000 (kode ruangan 1524020).

The material to be tested is stored in the staging trial room + 9.000 ( room code 1524020)

  - AREA PHARMA 2 LINE 6 +9.000

    - Setelah proses penimbangan bahan baku tablet yang akan diproduksi di area pharma 2 line 6, bahan baku dimasukkan ke dalam ruangan staging after weighing tablet pharma 2 line 6 +9000 (kode ruangan 1523004).

After weighing the tablet raw materials that will be processed in  pharma 2 line 6 area, the raw materials are stored in the staging room after weighing tablets pharma 2 line 6 +9.000 ( room code 152004)

    - Produk antara tablet pharma 2 line 6 yang dikarantina disimpan diruangan quarantine intermediate tablet pharma 2 line 6 +9.000 (kode ruangan 1523010).

Intermediate products of Pharma 2 line 6 tablets that are quarantined are stored in the intermediate quarantine room for pharma 2 line 6 tablets ( room code 1523010

    - Produk ruahan tablet pharma 2 line 6 dikarantina di quarantine bulk tablet pharma 2 line 6 +9.000 (kode ruangan 1523018).

Bulk production of pharma 2 line 6 tablets  are quarantined in quarantine bulk tablet pharma  2 line 6 + 9.000 (room code 1523018 )

    - Bahan kemas primer yang siap untuk proses pengemasan untuk produk tablet di area pharma 2 line 6 di simpan didalam ruangan staging primary packing tablet pharma 2 line 6 +9.000 (kode ruangan 1523027).

Primary packaging materials that are ready for the packaging process for tablet production in the Pharma 2 line 6 area are stored in the staging room primary packing tablets pharma 2 line 6 + 9.000 (room code 1523027)

    - Penyimpanan bahan kemas botol untuk dry sirup disimpan Staging Bottle +9.000 (kode ruangan 1523019).

Bottle packaging materials for dry syrup are stored in the staging bottle +9.000 (room code 1523019)

    - Setelah proses penimbangan bahan baku, kemudian bahan baku diserahkan ke produksi, selama proses menunggu untuk di produksi, bahan baku disimpan di ruangan staging after weighing oral powder non BL2 +9.000 (kode ruangan 1523035).

After the process of weighing raw materials, then the raw materials are handed over to production, during waiting to be processed, the raw materials are stored in room staging after weighing oral powder non BL2 +9.000 ( room code 1523035 )

    - Bahan kemas primer untuk produk oral powder disimpan dalam ruangan staging primary packaging oral powder non BL2 +9.000 (kode ruangan 1523048).

Primary packaging materials for the production of oral powders are stored in staging primary packaging oral powder non BL2 + 9.000 ( room code 1523048 )

    - Produk ruahan oral power karantina disimpan di quarantine bulk oral powder non BL2 +9.000 (kode ruangan 1523048).

Quarantine oral powder bulk products are stored in bulk quarantine oral powder non BL2   +9.000 ( room code 1523048)

  - AREA SIMPLICIA WAREHOUSE HERBAL +0.000

    - Bahan baku simplisia yang dikarantina, disimpan dalam area Sampling & Quarantine Simplicia +0.000 (kode ruangan 1711005).

Quarantine simplisia raw materials are stored in the simplicia sampling & quarantine area +0.000 (room code 1711005)

    - Bahan baku simplisia yang sudah diluluskan, disimpan di ruang Simplicia Warehouse +0.000 (kode ruangan 1711007).

Simplicia raw materials that have been released are stored in the room Simplicia Warehouse +0.000 (room code 1711005)

    - Bahan baku simplisia dan ektstrak yang di reject, disimpan di area Reject Simplicia +0.000 (kode ruangan 1711006).

The rejected simplicia raw materials and extract are stored in the rejected simplicia and extract area +0.000 ( room code 1711006 )

  - AREA SIMPLISIA BERSIH +0.000

- Bahan baku ekstrak kering yang dikarantina, disimpan dalam area gudang simplisia bersih (kode ruangan 1711024), dengan tanda pembatas/stand barrier dan label

Quarantined dry extract raw materials, are stored in a clean simplicia warehouse area (room code 1711024), with clear boundaries and label.

- Bahan baku ekstrak kering yang sudah diluluskan, disimpan dalam area gudang simplisia bersih (kode ruangan 1711024)

Released dry extract raw materials, are stored in a clean simplicia warehouse area (room code 1711024)

  - AREA WEIGHING SIMPLICIA HERBAL +0.000

    - Simplisia bersih yang diterima dari washing & sortir dan menunggu proses penimbangan dsimpan di ruang Staging Clean Simplicia +0.000 (kode ruangan 1711024).

Clean Simplicia received from washing & sorting and waiting for the weighing process are stored in room staging clean simplicia +0.000 (room code 1711024)

  - AREA WEIGHING EKSTRAK HERBAL +0.000

    - Menerima Bahan baku ekstrak dan eksipien dari warehouse menuju area deduster material dan airlock material berdasarkan prosedur dengan nomor prosedur PWR-03-006.

Receive raw material extracts and excipients from the warehouse to the material deduster area and material airlock according to the procedure with the PWR-03-006

    - Bahan baku yang menunggu proses penimbangan disimpan diruangan staging before weighing +0.000 (kode ruangan 1716015).

Raw materials waiting for the weighing process are stored in the room staging before weighing +0.000 ( room code 1716015 )

    - Setelah proses penimbangan selesai dan menunggu proses produksi selanjutnya, bahan baku disimpan diruangan staging after weighing +0.000 (kode ruangan 1716020).

After the weighing process is complete and waiting for the next production process, the raw materials are stored in the room staging after weighing +0.000 (room code 1716020 )

    - Bahan baku sisa penimbangan disimpan dalam ruang Staging & Retained Raw Material +0.000 (kode ruangan 1716013) dan cold storage 2⁰C - 8⁰C +0.000 (kode ruangan 1714013A).

The remaining of weighing raw materials are stored in the room staging & retained raw material +0.000 ( room code 1716013 ) and cold storage 2 C – 8 C +0.000 (room code 1714013A0

  - AREA PACKAGING WAREHOUSE HERBAL +0.000

    - Bahan kemas yang dikarantina, disimpan dalam area Quarantine Packaging +0.000 (kode ruangan 1717010B), dengan pembatas dan label.

Quarantined packaging materials are stored in Quarantine area Packaging warehouse +0.000 (room code 17170108 ), with clear boundary and label.

    - Bahan Kemas yang sudah diluluskan disimpan dalam ruang Packaging Warehouse +0.000 (kode ruangan 1514012).

Released Packaging Materials are stored in Packaging Warehouse +0.000 ( room code 1514012 )

  - AREA SECONDARY PACKAGING WAREHOUSE +6.000

    - Bahan kemas sekunder yang dikarantina, disimpan didalam area Quarantine Secondary Packaging +6.000 (kode ruangan 1724006).

Quarantined secondary packaging materials are stored in secondary packaging +6.000 (room code 1724006 )

    - Bahan kemas botol plastik atau kaca yang dikarantina, disimpan didalam area Quarantine Bottle +6.000 (kode ruangan 1724003).

Quarantined plastic or glass bottle packaging materials are stored in the area Quarantine bottle +6.000 ( room code 1724003 )

    - Bahan kemas sekunder yang telah diluluskan disimpan didalam ruang Secondary Packaging Warehouse +6.000 (kode ruangan 1724005).

Secondary packaging material that have been released are stored in the room Secondary packaging warehouse +6.000 (room code 1724005 )

    - Bahan kemas botol plastik atau kaca yang telah diluluskan, disimpan didalam area Bottle Warehouse +6.000 (kode ruangan 1724004).

Plastic or glass bottle packaging materials that have been released are stored in an area Bottle warehouse +6.000 (room code 1724004 )

    - Bahan kemas sekunder, termasuk botol kaca atau plastik yang ditolak, disimpan didalam ruang Reject Packaging Material +6.000 (kode ruangan 1724012)

Rejected Secondary packaging materials including rejected glass or plastic bottles are stored in room reject packaging material +6.000 (room code 1724012)

  - AREA RETURNED/REJECT PRODUCT HERBAL +6000

Produk jadi reject atau kembalian Herbal disimpan di ruangan Reject Returned Product Herbal (kode ruangan 1724011)

Returned/Reject Herbal Products are stored in Rejected/Returned Product Herbal Room (room code 1724011)

  - AREA CODING HERBAL +6.000 (SPARE)

    - Bahan kemas yang menunggu proses penandaan, disimpan didalam area Staging Before Coding +6.000.

Packaging materials that are waiting for the coding process are stored in an area staging before coding +6.000

    - Setelah proses penandaan selesai dan menunggu proses pengemasan, bahan kemas  terlebih dahulu disimpan diruangan Staging After Coding +6.000 (kode ruangan 1724008).

After the coding process is complete and waiting for the packaging process ,the packaging materials are stored in the room staging after coding +6.000 ( room code 1724008 )

- Daftar Lampiran/ List of Attachment

| Lampiran Attachments | Kode Dokumen Document Code | Judul Dokumen Document Title | Tanggal Efektif Effective Date |
| --- | --- | --- | --- |
| - | - | - | - |

- Sejarah Revisi dan Review/ Revision History and Review

| Kode Dokumen Document Code | Revisi Revision | Tanggal Efektif Effective Date | Perubahan Changes | Alasan Reasons |
| --- | --- | --- | --- | --- |
| PWR-02-002 | 00 | 09 Aug 2018 | - | Dokumen Baru |
| PWR-02-002 | 01 | 23 OCT 2020 | Perubahan pada point 2; 6.1.2; dan 6.2. Penambahan pada point 6.8; 6.9; 6.15; 6.16; 6.17; 6.18; 6.19; 6.20; dan 6.21. Penghapusan pada point 6.1.1; dan 6.7.2. | Menyesuaikan kondisi di lapangan. |
| PWR-02-002 | 02 |  | Perubahan pada point 6.7, 6.9, 6.15, 6.16, 6.19, 6.20, 6.21 Changes to points 6.7, 6.9, 6.15, 6.16, 6.19, 6.20, 6.21 |  |

- Dokumen Terkait/Related Document

| Kode Dokumen Document Code | Nama Dokumen Document Name |
| --- | --- |

- Daftar Distribusi Dokumen/ Distribution List of Document

Asli: SSDR

Controlled Copy:

| 1. | AMS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2. | SGBB |  |  |  |  |  |  |
| 3. | SPS |  |  |  |  |  |  |
| 4. | Penandaan dan bahan kemas |  |  |  |  |  |  |
| 5. | AMP I |  |  |  |  |  |  |
| 6. | Spv. Produksi 1 |  |  |  |  |  |  |
| 7. | AMP II dan herbal |  |  |  |  |  |  |
| 8. | Spv produksi 2 dan Herbal |  |  |  |  |  |  |