- Objective

To guarantee that the method for inspecting Magnesium Stearate raw materials is followed appropriately and in accordance with applicable spesifications.

- Scope

PT Kimia Farma Tbk. - Plant Banjaran Laboratory.

- Responsibility

The Quality Control Analyst is in charge of inspecting Magnesium Stearate raw materials in line with the job instructions.

- Definition

-

- Reference

Modifikasi USP 46 NF 41 (2023)

- Equipments and Materials

  - Equipments :

| 1. | Analytical Balance | 13. | Burette |
| --- | --- | --- | --- |
| 2. | Beaker Glass | 14. | Desiccator |
| 3. | Test Tube | 15. | Separator |
| 4. | Volumetric Flask | 16. | Weighing Bottle |
| 5. | Oven | 17. | Magnetic Stirrer |
| 6. | Measuring Pipette | 18. | Erlenmeyer flask |
| 7. | Inductively Coupled Plasma (ICP) | 19. | Conical Flask |  |
| 8. | Vortex | 20. | Waterbath |  |
| 9. | Measuring Cylinder | 21. | Nessler Tube |  |
| 10. | Crucible Tong | 22. | Filter Paper |  |
| 11. | Gas chromatography | 23. | Reflux apparatus |  |
| 12. | Column G16; 0,32 mm x 30 m; 0,5 µm |  |  |  |

  - Materials :

| 1. | Purified Water | 14. | P; 6 N Ammonium Hydroxide |
| --- | --- | --- | --- |
| 2. | CO2 - free Water | 15. | Dibasic Sodium Phosphate TS |
| 3. | 3 N; 1 N; 0,1 N LV; 0,02 N Hydrochloric Acid | 16. | Ether |
| 4. | Ammonium Chloride | 17. | 0,1 N Silver Nitrate |
| 5. | Butyl Alcohol | 18. | 0,1 M EDTA VS |
| 6. | 0,1 M Zinc Sulfate VS | 19. | Peroxide - free Ether |
| 7. | ICP Multi-element Standard Solution | 20. | 0,1 N Sodium Hydroxide VS |
| 8. | Dehydrated Alcohol | 21. | Ethanol |
| 9. | Eriochrome Black T | 22. | P; 2 N Nitric Acid |
| 10. | 0.02 N Sulfuric Acid | 23. | Barium Choride TS |
| 11. | Ammonium Carbonate TS | 24. | Bromothymol Blue TS |
| 12. | Boron Triflouride | 25. | n-Heptane |
| 13. | Saturated Sodium Hydroxide |  |  |

- Work Instruction

  - Description

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

Visually observe (Organoleptic).

  - Solubility

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Solubility In Water

- Dissolve 10 mg of sample substance with more than 100 mL of Purified Water in a beaker.

- Observe the description of the solution, there is clear solution.

- Solubility In Ethanol

- Dissolve 10 mg of sample substance with more than 100 mL of Ethanol in a beaker.

- Observe the description of the solution, there is clear solution.

- Solubility In Ether

- Dissolve 10 mg of sample substance with more than 100 mL of Ether in a beaker.

- Observe the description of the solution, there is clear solution.

  - Identification

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

      - Identification test A

- Weigh 5 grams of the sample substance, put it in an erlenmeyer flask.

- Add 50 mL of  Peroxide - free Ether, 20 mL of  HNO3 2 N, and 20 mL of Purified Water.

- Heat it on a Waterbath at 105C for 30 minutes.

- Allow to cool, then put into a separator.

- Shake the separator and allow the layers to separate.

- Separate the aqueous layer into another erlenmeyer flask.

- Extract the Ether layer with 4 mL of Purified Water.

- Shake the separator and allow the layers to separate.

- Separate the water layers and add them to the first water layer.

- Put it in another separator.

- Wash off the aqueous layer by adding 15 mL of Peroxide - Free Ether and shaking the separator.

- Separate the aqueous layer into a 50 mL volumetric flask.

- Add Purified Water to volume, shake until homogeneous (Solution A).

- Pipette 1 mL of Solution A into a test tube.

- Add Ammonium Chloride to the solution.

- A slightly hazy precipitate when neutralized with ammonium carbonate TS.

- On the subsequent addition of dibasic sodium phosphate TS, a white, crystalline precipitate, which is insoluble in 6 N ammonium hydroxide, is formed.

      - Identification test B

The retention times of the peaks corresponding to stearic acid and palmitic acid of the Sample solution correspond to those of the System suitability solution, as obtained in the test for Relative Content of Stearic Acid and Palmitic Acid.

  - Chloride

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Standard Solution

- Pipette 1.4 mL of 0.02 N HCl, put it in a Nessler tube.

- Sample Solution

- Pipette 10 mL of Solution A, put it in a nessler tube.

- Observation

- Into each Nessler tube containing the standard solution and the test solution, add purified water to a total volume of 30 - 40 mL.

- Also add 1 mL of 2 N HNO3 and 1 mL of 0.1 N AgNO3.

- Add Purified Water to a volume of 50 mL, mix until homogeneous.

Allow to stand for 5 minutes protected from direct sunlight.

- Observe the description of the solution, then compare the turbidity of the sample solution with the turbidity of the standard solution.

  - Sulfate

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Standard Solution

- Pipette 3 mL of 0.02 N H2SO4, put it in a nessler tube.

- Sample Solution

- Pipette 3 mL of Solution A, put it in a nessler tube.

- Observation

- Into each Nessler tube containing the standard solution and the test solution, add purified water to a total volume of 30 - 40 mL.

- Also add 1 mL of 3 N HCl and 1 mL of Barium Chloride 12%.

- Add Purified Water to a volume of 50 mL, mix until homogeneous.

- Allow to stand for 5 minutes.

- Observe the description of the solution, then compare the turbidity of the sample solution with the turbidity of the standard solution.

  - Assay

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Buffer

- Weigh 5.4 grams of Ammonium Chloride, put it in a beaker glass.

- Add 20 mL of Ammonium Hydroxide.

- Dilute with 80 mL of Purified Water, stir until homogeneous.

- Inspection

- Weigh 500 mg of the sample substance, put it in an erlenmeyer flask.

- Add 50 mL of the mixture of Butyl Alcohol : Alcohol Dehydrate (1:1); 5 mL Ammonium Hydroxide; 3 mL of Buffer; 30.0 mL of 0.1 M Edetate Sodium VS; and 2 drops of Eriochrome Black T.

- Heat at a temperature of 45 - 50oC until the solution is clear, allow to cool.

- Titrate the excess Edetate Sodium with 0.1 M Zink Sulfate VS until the solution color changes from blue to violet.

- Perform a blank titration.

Each mL of 0.1 M Edetate Sodium VS is equivalent to 2.431 mg of Magnesium.

    - Calculation :

C=Vb -Vu × M0,1 × 2,431Wu × 100%(100% -SP) × 100%

Note:

C	=	Assay of Magnesium (%)

Vb	=	Volume of Zinc Sulfate 0.1 M VS for titration of blank solution (mL)

Vu	=	Volume of Zink Sulfate 0.1 M LV for titration of sample solution (mL)

M	=	Zink Sulfate VS Molarity (M)

0,1	=	Theoretical molarity of Zink Sulfate VS (M)

2,431	=	Equivalence of 1 mL 0,1 M EDTA to Magnesium (mg)

Wu	=	Weight of sample substance weighed (mg)

SP	=	Loss on Drying (%)

  - Acidity or Alkalinity

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Weigh 1 gram of the sample substance, put it in a conical flask.

- Add 20 mL of CO2-free water, boil in a water bath for 1 minute.

- Continuous shaking, cool, and filter.

- Pipette 10 mL of the filtrate, then add 0.05 mL of Bromtimol Blue LP.

- If the solution is yellow, titrate the solution with 0.1 N NaOH VS until it becomes blue.

- If the solution is blue, titrate the solution with 0.1 N HCl VS until it becomes yellow.

- Record the volume of the titrant.

  - Loss On Drying

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Heat the weighing bottle at a temperature of 105oC in the Oven, for 30 minutes.

- Allow to cool the weighing bottle in a desiccator for 5 – 15 minutes and weigh thoroughly (Wo).

- If the test substance is big crystals, mill it quickly to a particle size of ± 2 mm.

- Weigh carefully 1 g of the test substance, put it in a weighing bottle that has been weighed (Wa).

- Put it in the oven at 105oC.

- Dry for 3 hours.

- Cool in a desiccator for 5 - 15 minutes before weighing (Wb).

    - Calculation :

SP= Wa-WbWa-Wo × 100%

Note:

SP	= 	Loss on drying (%)

Wa	= 	Weight of weighing bottle + sample substance before drying (grams)

Wb	= 	Weight of weighing bottle + sample substance after drying (grams)

Wo	= 	Weight of empty weighing bottle (grams)

  - Heavy Metals

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

- Examination of Lead (Pb), Cadmium (Cd), dan Nickel (Ni)

- Nitric Acid 2,5% Solution

- Fill a 1000 mL beaker halfway with 500 mL of fresh water.

- Slowly pour in 35,71 mL of Nitric Acid  and mix until smooth.

- Add Purified Water until volume solution 1000 ml; stir until homogeneous.

- Multi-Element Standard Solution

- Pipette 10 mL ICP Multi-element Standard Solution IV 100 ppm, placed in a 100 mL volumetric flask.

- To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Filter using a 0.2 µm syringe filter.

(Concentration of Pb, Cd, Ni: 10 ppm)

- Standard Series Solutin

- Pipette an amount of standard solution 10 ppm  according to the table below, put in a 100 ml measuring flask.

| Concentration (ppm) | Pipetting Volume (mL) | Measuring Flask (mL) |
| --- | --- | --- |
| 0.010 | 0.1 | 100 |
| 0.050 | 0.5 | 100 |
| 0.150 | 1.5 | 100 |
| 0.250 | 2.5 | 100 |
| 0.500 | 5 | 100 |
| 1.000 | 10 | 100 |

- To indicate the limit; add 2.5% Nitric Acid solution and mix until homogenous.

- Filter using a 0.2 µm syringe filter.

- Sample Solition

- Weight 0,5 g sample Magnesium Stearat, put into  crucible porcelain.

- Incandescent in a furnace at a temperature of 475 - 500 oC for 20 minutes.

- Allow to cool until room temperature.

- Add 3 drops of Nitric Acid.

- Incandescent in a furnace at a temperature of 475 - 500 oC for 20 minutes.

- Dissolve the residue in 1 ml of a mixture Nitric Acid P and Water (1 : 1).

- Put the digestion solution into a 50 mL measuring flask quantitatively.

- Dilute with 2.5% nitric acid to the mark, shake until homogeneous.

- Filter with a 0.45 µm membrane filter, collect the filtrate.

- System Suitability Test Solution

Read time			: 8 s

Rinse time			: 45 s

RF Power			:1.2 kW

Stabilization time		: 15 s

Viewing mode		: Axial

Nebulizer flow		: 0.7 L/min

Plasma			: 12 L/min

Aux Flow			: 1 L/min

Lead Wavelength		 	: 220.353 nm

Replikasi			: 3 Replication

- Acceptance Criteria System Suitability Test

| No | Category | Acceptance Criteria |
| --- | --- | --- |
| 1 | Correlation coefficient | r ≥ 0.9900 |
| 2 | % Error | % Error ≤ 20% |
| 3 | QC Check (Multi elements middle standard) | % recovery : 80 – 120 % |

Notes : Perform the middle standard after injecting the standard series and at the end of injecting the sample.

    - Perhitungan:

C ꓿Intensitas ־ Intercept Slopex VolMassa x FP

Notes :

Cu		: Calculated heavy metal content (ppm)

Intensitas	: The intensity of the test solution

Intercept	: The intercept of the standard series

Slope		: The slope of the standard series

Vol		: Initial volume (50 mL)

Massa		: Weight of sample test (gram)

Fp 		: Dilution factor (1)

  - Relative Content of Stearic Acid and Palmitic Acid (Examination carried out with the assistance of an external laboratory)

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

      - Boron Trifluoride solution

- Weigh 14 g of Boron Triflouride, put into 100 mL volumetric flask.

- Dissolve and dilute to volume using Methanol, shake until homogenous.

      - System Suitability solution

- Weigh 50 mg Stearic acid and 50 mg Palmitatic acid, put into small Erlenmeyer flask connected into reflux condenser.

- Boron Trifluoride solution, reflux for 10 minutes until the solids have dissolved.

- Add 4 mL of n-Heptane for chromatography through the condenser, and reflux again for 10 minutes.

- Cool, then add 20 mL of saturated Sodium Hydroxide, shake, and allow the layers to separate.

- Flow the layers of n-Heptane through 0.1 g of Anhydrous Sodium Sulfate (previously washed with n-Heptane for chromatography), collect to suitable vial.

- Pipette 1 mL of solution, put into 10 mL volumetric flask.

- Dilute to volume using n-Heptane (for chromatography).

      - Sample solution

- Weigh 100 mg sample, put into small Erlenmeyer flask connected into reflux condenser.

- Add 5 mL Boron Trifluoride solution, reflux for 10 minutes until the solids have dissolved.

- Add 4 mL of n-Heptane for chromatography through the condenser, and reflux again for 10 minutes.

- Cool, then add 20 mL of saturated Sodium Hydroxide, shake, and allow the layers to separate.

- Flow the layers of n-Heptane through 0.1 g of Anhydrous Sodium Sulfate (previously washed with n-Heptane for chromatography), collect to suitable vial.

- Pipette 1 mL of solution, put into 10 mL volumetric flask.

- Dilute to volume using n-Heptane (for chromatography).

      - Chromatography system

Column			: G16; 0,32 mm x 30 m; 0,5 µm

Injector temp.		: 220°C

Detector temp.		: 260°C

Column temp.		:

| Initial Temperature (°) | Temperature Ramp (°/min) | Final Temperature (°) | Hold Time at Final Temperature (min) |
| --- | --- | --- | --- |
| 70 | - | 70 | 2 |
| 70 | 5 | 240 | 5 |

Detector			: Flame ionization

Carrier gas			: Helium

Flow rate			: 2,4 mL/min

Injection volume		: 1 µl

      - System suitability requirement

| Parameter | Acceptance criteria |
| --- | --- |
| Resolution | ≥ 5.0% between Methyl Palmitate and Methyl Stearate |
| Area | %RSD peak of system suitability solution ≤ 3.0% |
| Area ratio | %RSD peak of system suitability solution ≤ 1.0% |
| Retention time | 0.9 – 1.0 between Methyl Palmitate and Methyl Stearate |
| Tailing Factor | Tailing factor peak of standard solutions : ≤ 2.0 |
| Total Plate Count | Total Plate Count : ≥ 2000 |

    - Calculation:

C= LiLt × 100

Note :

| C | = Relative content assay of Stearic acid/Palmitic acid (%) |
| --- | --- |
| Li | = Area Methyl Stearate/Methyl Palmitate of sample solution |
| Lt | = Area total Fatty acid of sample solution |

  - Relative Content of Stearic Acid and Palmitic Acid

    - Requirement :

Magnesium Stearate Raw Material specification are met a requirement (S-QC-01-071).

    - Procedure :

Follow the Microbial Enumeration Tests Procedure when performing inspections.

  - List of Attachment

| Attachments | Document Code | Document Title | Effective Date |
| --- | --- | --- | --- |
| - | - | - | - |

  - Revision and Review History

| Document Code | Revision | Effective Date | Changes | Reasons |
| --- | --- | --- | --- | --- |
| I-QC-01-015(E) | 00 |  | - | New Document |

  - Distribution List of Document

Original : SSDR

Controlled Copy :

- AMPM

- SPBB