- Objective

To show specific description of Molnupiravir raw material to be used as a reference on inspection of the material.

- Scope

Explain the raw material Molnupiravir specifications used in the production process at PT. Kimia Farma Tbk. Banjaran Plant.

- Material Specification

Synonyms	:	-

Molecular formula	: C13H19N3O7

Molecular weight	: 	329,309 g/mol

Material code	: 	31001290

| No | Category | Unit | Acceptance Criteria | Reference |
| --- | --- | --- | --- | --- |
| 1 | Description |  | White to off-white, crystalline powder | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 2 | Identification |  |  |  |
|  | Infrared absorption |  | The infrared absorption spectrum of the sample substance shows a maximum only at the same wavelength as that of the standard substance. | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | HPLC Methode |  | Retention time of main peak chromatogram sample solution according to standard solution obtained as Assay. | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 3 | Water Content | % | Not more than 0.5 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 4 | Optical Rotation | deg | Between -12.0 and -6.0 | In House Method (Adopted from API manufacturer Shanghai Desano) |

| No | Category | Unit | Acceptance Criteria | Reference |
| --- | --- | --- | --- | --- |
| 5 | # Residue on Ignition | % | #### Not more than 0.1 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 6 | Heavy Metal | ppm | Not more than 10 | Modification USP 46 NF 41 (2023) |
| 7 | # Assay (calculated on dried and solvent free basis ) | % | ### 98,0 - 102,0 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 8 | Impurity |  |  |  |
|  | Impurity A (Ph. Int draft impurity D) | % | ### Not more than 0.06 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Impurity B (Ph. Int draft impurity A) | % | ### Not more than 0.50 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Impurity C (Ph. Int draft impurity L) | % | ### Not more than 0.06 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Impurity D (Ph. Int draft impurity F) | % | ### Not more than 0.06 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | (Any other individual impurity) | % | ### Not more than 0.06 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Total Impurity | % | ### Not more than 1.0 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 9 | Residual Solvent - Methode I |  |  |  |
|  | Isopropyl Alcohol | % | Not more than 0.5000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Dichloromethane | % | Not more than 0.0600 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Methanol | % | Not more than 0.3000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Isopropyl Acetate | % | Not more than 0.5000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Pyridine | % | Not more than 0.0200 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Ethyl Acetate | % | Not more than 0.5000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Benzene | % | Not more than 0.0002 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 10 | Residual Solvent - Methode II |  |  |  |
|  | N,N-dimethylformamide | % | Not more than 0.0880 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | Triethylamine | % | Not more than 0.5000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
|  | 2-methyl Tetrahydrofuran | % | Not more than 0.5000 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 11 | Impurity 11 | % | Not more than 0.06 | In House Method (Adopted from API manufacturer Shanghai Desano) |
| 12 | Microbial Enumeration Tests |  |  |  |
|  | The Total Aerobic Microbial Count | cfu/g | Not lower than 103 (2,000) | USP 46 NF 41 (2023) |
|  | The Total Combined Molds and Yeasts Count | cfu/g | Not lower than 102 (200) | USP 46 NF 41 (2023) |

- Revision and Review History

| Document Code | Revision | Effective Date | Changes | Reasons |
| --- | --- | --- | --- | --- |
| S-QC-01-280 | 00 |  | - | New Document |

- Distribution List of Document

Original : SSDR

Controlled Copy :

- AMPM

- SPBB