- Objective

To show specific description of Magnesium Stearate raw material to be used as a reference on inspection of the material.

- Scope

Explain the raw material Magnesium Stearate specifications used in the production process at PT. Kimia Farma Tbk. Banjaran Plant.

- Material Specification

Synonyms	:	-	Magnesium Octadecanoate

- Magnesium Salt

- Octadecanoic Acid

- Stearic Acid Magnesium Salt

- Octadecanoic Acid Magnesium Salt

- Dibasic Magnesium Stearate

- Magnesium Distearat

Molecular formula	: C36H70MgO4

Molecular weight	: 591,257 gram/mol

Material code	: 32000073, 32000174

| No | Category | Unit | Acceptance Criteria | Reference |
| --- | --- | --- | --- | --- |
| 1 | Description |  |  |  |
|  | Shape |  | Very fine powder, light | USP 46 NF 41 (2023) |
|  | Color |  | White | USP 46 NF 41 (2023) |
|  | Other characteristic |  | Slippery to touch | USP 46 NF 41 (2023) |
| 2 | Solubility |  |  |  |
|  | In water |  | Insoluble | USP 46 NF 41 (2023) |
|  | In ethanol |  | Insoluble | USP 46 NF 41 (2023) |
|  | In ether |  | Insoluble | USP 46 NF 41 (2023) |
| 3 | Identification |  |  |  |
|  | Identification test A |  | A white crystal precipitate is formed which is insoluble in 6 N Ammonium Hydroxide | USP 46 NF 41 (2023) |
|  | Identification test B |  | The retention times of the peaks corresponding to stearic acid and palmitic acid of the Sample solution correspond to those of the System suitability solution, as obtained in the test for Relative Content of Stearic Acid and Palmitic Acid | USP 46 NF 41 (2023) |
| 4 | Chloride | mL | Not more than 1.4 ml of 0.02 N Hydrochloric Acid (0.1%) | USP 46 NF 41 (2023) |
| 5 | Sulfate |  | Not more than 3.0 ml of 0.02 M Sulfuric Acid (1.0%) | USP 46 NF 41 (2023) |
| 6 | Assay, calculated on the dried basis | % | 4,00 – 5,00 | USP 46 NF 41 (2023) |
| 7 | Acidity or Alkalinity | ml | Not more than 0.05 mL of 0,1 N HCl or 0.1 N NaOH | USP 46 NF 41 (2023) |
| 8 | Loss on Drying | % | Not more than 6.0 | USP 46 NF 41 (2023) |
| 9 | Heavy Metals |  |  |  |
|  | Limit of Cadmium | ppm | Not more than 3 | Modifikasi USP 46 NF 41 (2023) |
|  | Limit of Lead | ppm | Not more than 10 | Modifikasi USP 46 NF 41 (2023) |
|  | Limit of Nickel | ppm | Not more than 5 | Modifikasi USP 46 NF 41 (2023) |
| 10 | Relative content of Stearic Acid and Palmitic Acid |  |  |  |
|  | Stearate peak | % | Not less than 40 | USP 46 NF 41 (2023) |
|  | The sum of the Stearate and Palmitate peaks (from total areas of all the fatty acids) | % | Not less than 90 | USP 46 NF 41 (2023) |
| 11 | Microbial Limit |  |  |  |
|  | Total Aerobic Microbial Count | cfu/g | Not more than 103 (2000) | USP 46 NF 41 (2023) |
|  | Yeast And Molds Count | cfu/g | Not more than 5 x 102 (500) | USP 46 NF 41 (2023) |
|  | Salmonella sp |  | Negative | USP 46 NF 41 (2023) |
|  | Escherichia coli |  | Negative | USP 46 NF 41 (2023) |

- Revision and Review History

| Document Code | Revision | Effective Date | Changes | Reasons |
| --- | --- | --- | --- | --- |
| S-QC-01-280 | 00 |  | - | New Document |

- Distribution List of Document

Original : SSDR

Controlled Copy :

- AMPM

- SPBB