- Objective

To guarantee that the method for inspecting Croscarmellose Sodium raw materials is followed appropriately and in accordance with applicable spesifications.

- Scope

PT Kimia Farma Tbk. Plant Banjaran Laboratory.

- Responsibility

The Quality Control Analyst is responsible for inspecting Croscarmellose Sodium raw materials in line with the working instructions.

- Definition

-

- Reference

USP 46 NF 41 (2023)

- Equipments and Materials

  - Equipments :

| 1. | Analytical Balance | 14. | Crucible |
| --- | --- | --- | --- |
| 2. | Beaker Glass | 15. | Measuring Cylinder |
| 3. | Test Tube | 16. | Buchner Funnel |
| 4. | pH Meter | 17. | Drop Pipette |
| 5. | Oven | 18. | Glass-Stoppered Conical Flask |
| 6. | Weighing Bottle | 19. | Burette |
| 7. | Desiccator | 20. | Magnetic Stirrer |
| 8. | Furnace | 21. | Hot Plate |
| 9. | Measuring Pipette | 22. | Measuring Cylinder with Lid |
| 10. | Filter Paper | 23. | Crucible Tong |
| 11. | Autotitrator | 24. | Water bath |
| 12. | Spectrophotometer UV-Vis | 25. | Platinum wire |
| 13. | Spectrophotometer Infrared |  |  |

  - Materials :

| 1. | Purifed Water | 10. | Sodium Chloride Solution (1 in 10) |
| --- | --- | --- | --- |
| 2. | Ethanol | 11. | 0.1 N Natrium Hidroksida Volumetric Standard |
| 3. | Methylene Blue (1 : 250,000) | 12. | 0.1 N Hydrochloric Acid Volumetric Standard |
| 4. | m-violet cresol TS | 13. | Sulfuric acid |
| 5. | 1-Naphtol | 14. | 2,7-Dihydroxynaphthalene |
| 6. | 0,1 N Silver nitrate | 15. | Glycolate acid |
| 7. | Glacial Acetic Acid | 16. | Acetone |
| 8. | Potassium Bromide P | 17. | 15% Potassium Carbonate |
| 9. | Potassium Pyroantimonate TS |  |  |

- Work Instruction

  - Description

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

Observe visually (Organoleptic).

  - Solubility

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

- Solubility in Water

- Dissolve 1 gram of the sample substance with 30 - 100 mL of Purified Water in a beaker glass.

- Observe the description of the solution.

- Solubility in Ethanol

- Dissolve 10 mg of the sample substance with 100 mL of Ethanol in a beaker glass.

- Observe the description of the solution.

- Solubility in Ether

- Dissolve 10 mg of the sample substance with 100 mL of Ether in a beaker glass.

- Observe the description of the solution.

- Solubility in Methanol

- Dissolve 10 mg of the sample substance with 100 mL of Methanol in a beaker glass.

- Observe the description of the solution.

- Solubility in Aceton

- Dissolve 10 mg of the sample substance with 100 mL of Aceton in a beaker glass.

- Observe the description of the solution.

- Solubility in Chloroform

- Dissolve 10 mg of the sample substance with 100 mL of Chloroform in a beaker glass.

- Observe the description of the solution.

  - Identification

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

  - Infrared identification test

- Standard substance

- Mix and grind 8 mg of Crosscarmellose Sodium working standard with 400 mg of dry Potassium Bromide in a mortar and pestle.

- Make Potassium Bromide pellets.

- Sample substance

- Mix and grind 8 mg of sample with 400 mg of dry Potassium Bromide in a mortar and pestle.

- Make Potassium Bromide pellets.

- Measurement

- Use an IR spectrophotometer to do "scanning" at a wavelength of 400 - 4000 nm.

- Compare the spectra of the test substance with the standard substance.

- Observe the two spectra, the infrared absorption spectrum of the test substance shows a maximum only at the same wavelength as the standard substance.

  - Identification test B

## Mix 1 g of sample with 100 mL of Methylene Blue Solution (1 in 250,000).

## Stir the mixture and allow it  to settle.

## Observe visually.

  - Identification test C

- Dissolve a residue from the Residue on ignition test using 2 mL Purified water.

- Add 2 mL of 15% Potassium Carbonate, heat to boiling : No precipitate is formed.

- Add 4 mL of Potassium Pyroantimonate TS, heat to boiling.

- Cool using ice water, if necessary rub the inside of the test tube with glass rod : A dense precipitate is formed.

  - Residue of ignition

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

- Incandescent crucible at a temperature of 600 ± 50oC in the Furnace, for 30 minutes.

- Cool the crucible in a desiccator before weighing it (Wo).

- Weigh 1 g of sample, put into crucible (Wa).

- Add 1 mL of concentrated sulfuric acid.

- Heat until sample substance is carbonized perfectly, then allow to cool.

- Moisten the residue using 1 mL of concentrated Sulfuric Acid.

- Reheat until the white steam has dissipated.

- Incandescent inside furnace at 600 ± 50oC until the charcoal burns out.

- Cool in  desiccator, then weigh (Wb).

- If the amount of residue obtained exceeds the specified limit, moisten the residue again with 1 mL of Sulfuric Acid P.

- Reheat until the white steam is lost.

- Incandescent in the Furnace at a temperature of 600 ± 50 oC for 30 minutes.

- Allow to cool in desiccator, then weigh (Wb).

    - Calculation :

SPn= Wb-WoWa-Wo ×100%(100%-SP) x 100%

Note :

SPn	= 	Residue of ignition (%)

Wa	= 	Weight of crucible + test substance before ignition (grams)

Wb	= 	 Weight of crucible + sample substance after ignition (grams)

Wo	= 	Weight of empty crucible (grams)

SP    =  Loss on Drying (%)

  - Sodium Chloride and Sodium Glycolate

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

- Sodium Chloride

      - Weigh 5 g of sample, put it into a 250 ml Erlenmeyer.

      - Add 50 mL of Purified water and 5 mL of Hydrogen Peroxide 30%, and heat on a water bath for 20 minutes, stirring occasionally.

      - Cool, and add 100 mL of Purified water and 10 mL of 10% Nitric acid.

      - Titrate with 0.05 N Silver Nitrate using Eosin as a indicator until color changes to pink.

Calculation :

C= Vu x N x 584,4Wu x (100%-SP)

Note :

| C = | Sodium chloride assay (%) |
| --- | --- |
| 584,4= | Equivalence factor for Sodium Chloride |
| V = | Volume of 0,05 N Silver Nitrate (mL) |
| SP = | Loss on drying (%) |
| Wu = | Weight of sample being weighed (mg) |

- Sodium Glycolate

      - 2,7 Dihydroxynaphtalene Solution

- Weigh 50 mg 2,7-Dihydroxynapthalene, put into a 500 ml dark reagent bottle.

- Dissolve and dilute with 500 ml Sulfuric Acid P, shake until homogeneous.

- Solution can be used for 30 days.

(If the solution is yellow it means the solution is stable, and if the solution changes color to dark then make a new solution with Sulfuric Acid of a different origin)

  - Standard stock solution

- Weigh 100 mg of Glycolic acid  to a 100 mL volumetric flask.

- Dissolve with 50 mL of Purified Water, ultrasonic for 15 minutes.

- Cool until room temperature.

- Add Purified Water to volume, shake until homogeneous.

(Standard Stock Solution Concentration: 1 mg/ml)

  - Standard solution A

- Pipette 1 mL of Standard stock solution into a 100 mL volumetric flask.

- Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid, stir gently.

- Dilute with Acetone to volume, shake until homogeneous.

- Pipette 2 ml solution, put into a 25 ml volumetric flask.

- Heat the volumetric flask in a water bath at 70°C for 20 minutes for removing Acetone solution. Make sure the mouth of the volumetric flask is not closed.

- Remove from the waterbath and cool to room temperature.

- Add 5 ml 2,7 Dihydroxynapthalene, stir gently.

- Then add back 15 ml of 2,7 Dihydroxynapthalene, stir gently.

- Cover the mouth of the volumetric flask with aluminum foil.

- Reheat the volumetric flask in the water bath at 70°C for 20 minute.

- Remove from the waterbath and cool to room temperature.

- Dilute with Sulfuric Acid P until volume.

- Shake until homogeneous.

(Concentration of standard solution A: 0.0008 mg/ml)

  - Standard solution B

- Pipette 2 mL of Standard stock solution into a 100 mL volumetric flask.

- Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid, stir gently.

- Dilute with Acetone to volume, shake until homogeneous.

- Pipette 2 ml solution, put into a 25 ml volumetric flask.

- Heat the volumetric flask in a water bath at 70°C for 20 minutes for removing Acetone solution. Make sure the mouth of the volumetric flask is not closed.

- Remove from the waterbath and cool to room temperature.

- Add 5 ml 2,7 Dihydroxynapthalene, stir gently.

- Then add back 15 ml of 2,7 Dihydroxynapthalene, stir gently.

- Cover the mouth of the volumetric flask with aluminum foil.

- Reheat the volumetric flask in the water bath at 70°C for 20 minute.

- Remove from the waterbath and cool to room temperature.

- Dilute with Sulfuric Acid P until volume.

- Shake until homogeneous.

(Concentration of standard solution A: 0.0016 mg/ml)

  - Standard solution C

- Pipette 3 mL of Standard stock solution into a 100 mL volumetric flask.

- Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid, stir gently.

- Dilute with Acetone to volume, shake until homogeneous.

- Pipette 2 ml solution, put into a 25 ml volumetric flask.

- Heat the volumetric flask in a water bath at 70°C for 20 minutes for removing Acetone solution. Make sure the mouth of the volumetric flask is not closed.

- Remove from the waterbath and cool to room temperature.

- Add 5 ml 2,7 Dihydroxynapthalene, stir gently.

- Then add back 15 ml of 2,7 Dihydroxynapthalene, stir gently.

- Cover the mouth of the volumetric flask with aluminum foil.

- Reheat the volumetric flask in the water bath at 70°C for 20 minute.

- Remove from the waterbath and cool to room temperature.

- Dilute with Sulfuric Acid P until volume.

- Shake until homogeneous.

(Concentration of standard solution A: 0.0024 mg/ml)

  - Standard solution D

- Pipette 4 mL of Standard stock solution into a 100 mL volumetric flask.

- Add Purified water to make 5 mL and add 5 mL of Glacial acetic acid, stir gently.

- Dilute with Acetone to volume, shake until homogeneous.

- Pipette 2 ml solution, put into a 25 ml volumetric flask.

- Heat the volumetric flask in a water bath at 70°C for 20 minutes for removing Acetone solution. Make sure the mouth of the volumetric flask is not closed.

- Remove from the waterbath and cool to room temperature.

- Add 5 ml 2,7 Dihydroxynapthalene, stir gently.

- Then add back 15 ml of 2,7 Dihydroxynapthalene, stir gently.

- Cover the mouth of the volumetric flask with aluminum foil.

- Reheat the volumetric flask in the water bath at 70°C for 20 minute.

- Remove from the waterbath and cool to room temperature.

- Dilute with Sulfuric Acid P until volume.

- Shake until homogeneous.

(Concentration of standard solution A: 0.0032 mg/ml)

  - Sample solution

      - Weigh 500 mg of sample, put into 100 mL volumetric flask.

      - Wet thoroughly with 5 mL of Glacial acetic acid and 5 mL of Purified water. Stir with stirring rod for 15 minutes.

      - Slowly add 50 mL of Acetone while stirring.

      - Add 1 g of Sodium chloride, and stir for several minutes (to ensure complete precipitation of the Carboxymethylcellulose).

      - Filter using filter paper (soft and open-textured) that previously wetted with Acetone, collect the filtrate in a 100 mL volumetric flask.

      - Wash the precipitate using 30 ml of Acetone, then add Acetone to volume, shake until homogeneous.

- Pipette 2 ml solution, put into a 25 ml volumetric flask.

- Heat the volumetric flask in a water bath at 70°C for 20 minutes for removing Acetone solution. Make sure the mouth of the volumetric flask is not closed.

- Remove from the waterbath and cool to room temperature.

- Add 5 ml 2,7 Dihydroxynapthalene, stir gently.

- Then add back 15 ml of 2,7 Dihydroxynapthalene, stir gently.

- Cover the mouth of the volumetric flask with aluminum foil.

- Reheat the volumetric flask in the water bath at 70°C for 20 minute.

- Remove from the waterbath and cool to room temperature.

- Dilute with Sulfuric Acid P until volume.

- Shake until homogeneous.

  - Measurement

      - Determine the absorbance of solution at 540 nm using Spectrofotometer UV-Vis at 540 nm.

      - Follow the Spectrofotometer UV-Vis Equipment Work Instructions when performing inspections

Calculate :

C= 12,9 x WsWu x (100%-SP)

Note :

| C = | Sodium Glycolate assay (%) |
| --- | --- |
| 12,9 = | Factor converting Glycolic acid to Sodium Glycolate |
| Ws = | Weight of standard being weighed is determined from the standard curve and the absorbance of the sample solution (mg) |
| SP = | Loss on drying (%) |
| Wu = | Weight of sample being weighed (mg) |

  - Content of Water-Soluble Material

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

      - Weight 10 g of sample (W2), put into 1000 mL of beaker glass.

      - Dispersed in 800 mL of purified water.

      - In the first 30 minutes, every 10 minutes the solution was stirred using a magnetic stirrer for 1 minute.

      - Allow to stand for an additional hour, or centrifuge, if necessary.

      - Decant 200 mL of the aqueous slurry into a buchner funnel lined with filter paper.

      - Put 150 mL of the filtrate into a 250 mL beaker glass that has been heated, cooled, and weighed (W3).

      - Concentrate on a hot plate to a small volume, but not to dryness.

      - Dry at 105oC for 4 hours.

      - Allow to cool in a desiccator for 5 – 15 minutes, weigh the residue (W1).

    - Calculation :

W1= Wb-Wo

W3= Wf-Wo

BLA= [100 × W1 × 800+ W2]W2 × W3 × [1-0,01 × b]

Note :

BLA	= 	Content of Water-Soluble Material (%)

W1	= 	Weight of the residue (g)

W2	= 	Weight of the sample substance being weighed (grams)

W3	= 	Weight of  the filtrate (grams)

b	= 	Loss on drying (%)

Wo	=   Weigh of empty breaker glass (g)

Wb	=	Weight of breaker glass + residue (g)

Wf	=	Weight of breaker glass + filtrat (g)

  - Degree of substitution

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

      - Weigh 1 g of sample, put it in a  glass-stoppered 500 mL conical flask.

      - Add 300 mL of Sodium Chloride Solution (1 in 10).

      - From burette add 25 mL  of 0.1 N Sodium Hydroxide Volumetric Standard.

      - Insert the stopper, and allow to stand for 5 minutes with intermittent shaking.

      - Add 5 drops of m-violet cresol TS and  from a burette add 15 mL 0,1 N Hydrochloric Acid  Volumetric Standard; cover the glass-stoppered conical flask  and shake.

      - If the solution is violet, add 0.1 N Hydrochloric Acid Volumetric Standard until the solution becomes yellow (shaking after each addition).

      - Titrate with 0.1 N Sodium Hydroxide Volumetric Standard to a violet endpoint.

    - Calculated :

Miliequivalents of base required for the neutralization of 1 g of Croscarmellose Sodium, on the dried basis :

M=V ×N ×100 %100% -SP

The Degree of Acid Carboxymethyl Substitution :

A=1150 × M[7102-412 × M-80 × C]

The Degree of Sodium Carboxymethyl Substitution :

S=162+58 × A × C[7102-80 × C]

Degree of Substitution :

DS=A+S

Note :

V		=	Titration Volume (mL)

N		=	Concentration of NaOH used as titrant (N)

M  	= 	The milliequivalent of base used to neutralize 1 gram of sample calculated by dry weight (mek)

C		= 	Residue of ignition percentage (%)

A		=	Carboxymethyl acid degree of substitution

S		= 	 Sodium carboxymethyl degree of substitution

DS		= 	 Total degree of substitution

  - Loss on drying

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

- Heat the weighing bottle at a temperature of 105oC in the Oven, for 30 minutes.

- Allow to cool the weighing bottle in a desiccator and weigh thoroughly (Wo).

- If the test substance is big crystals, mill it quickly to a particle size of ± 2 mm.

- Weigh carefully 1 - 2 g of the test substance, put it in a weighing bottle that has been weighed (Wa).

- Flatten the sample in the weighing bottle ± 5 mm – 10 mm.

- Put it in the oven at 105oC.

- Dry for 6 hours.

- Allow to cool in a desiccator for 5 - 15 minutes , then weighing (Wb).

    - Calculation :

SP= Wa-WbWa-Wo × 100%

Note :

SP		= 	Loss on drying (%)

Wa	= 	Weight of weighing bottle + sample before drying (grams)

Wb	= 	Weight of weighing bottle + sample after drying (grams)

Wo	= Weight of empty weighing bottle (grams)

  - pH

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

      - Weigh 1 g of sample, put it in a beaker glass.

      - Add 100 mL of purified water.

      - Stir for 5 minutes.

      - Follow the pH Meter Equipment Work Instructions when performing inspections.

  - Settling Volume

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

      - Put 75 mL of purified water into 100 mL of measuring cylinder with lid.

      - Add 3 x 0.5 g of sample, close and shake every time adding.

      - Add purified water until 100 mL, shake until homogenous.

      - Let it for 4 hours.

      - Noted the volume of the settled mass.

  - Microbial Enumeration Test

    - Requirement :

Croscarmellose Sodium Raw Material specification are met a requirement (S-QC-01-128).

    - Procedure :

Follow the Microbial Enumeration Tests Procedure when performing inspections.

- List of Attachment

| Attachments | Document Code | Document Title | Effective Date |
| --- | --- | --- | --- |
| - | - | - | - |

- Revision and History Review

| Document Code | Revision | Effective Date | Changes | Reasons |
| --- | --- | --- | --- | --- |
| I-QC-01-127 (E) | 00 |  | - | Dokumen Baru New Document |

- Distribution List of Document

Original : SSDR

Controlled Copy:

      - AMPM

      - SPBB